Parent Item/BOM Code: 102731               UPPER ASY;PCL;FINAL           EA

As Of: 04/30/20  Levels:                Rev:

PCO Number:               ID:             Domain:          Output: page

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

Parent  102731             UPPER ASY;PCL;FINAL                   EA

1       100105             X,PLUMBING ASSY,UPPER             2.0 EA Yes

,PCL

.2      00934              SEALANT; PIPE DOPE;ONE            0.0 EA

PINT CAN,RECTORSEAL #5

.2      02549              BUSHING;CONDENSER;CONV.           1.0 EA

STMR.1/2"X1/4X1/8

..3     RB01210            7/8HEX YELLOW BRASS BAR         0.078 FT

.2      02594              BUSHING;HEX;3/8x1/4;BR            1.0 EA

.2      06188              FTG.;COMP.TEE 1/4Tx1/4x           1.0 EA

1/4MPT;BR.

.2      06194              FTG;COMP.1/4Tx3/8MPT 90'          1.0 EA

.2      06216              FTG.;COMP.1/4Tx1/4MPT             1.0 EA

.2      100223             TEE;1 IN X 1/2 IN X 1 IN          1.0 EA

BLACK

.2      101394             FTG.;COMP;TEE;1/8MPT X            1.0 EA

1/4T X 1/4T

.2      101924             NIPPLE1.00NPTx 5.50               1.0 EA

SCH 40;BLK IRN TBE

.2      102078             TUBE;FORMED;PINCH VALVE           1.0 EA Yes

/DRAIN;PCL

..3     00728              TUBING;Cu; SEAMLESS.25          0.708 FT

ODX.028W-10'SOFT STR

.2      104464             VALVE;PINCH;1 IN                  1.0 EA

.2      14304              NIPPLE .25NPTxCLOSE SCH           1.0 EA

40; BRASS TBE

.2      14369              NIPPLE1.00NPTxCLOSE; SCH          1.0 EA

40; BLK IRON TBE

.2      14553              NOZZLE;SPRAY;DRAIN MANIF          1.0 EA

;GD1-1/8 GG;FULL JET;BRS

.2      15462              ORIFICE;FLOW CONTROLLER;          1.0 EA

1/8 GPM

.2      20245              TEE;MALE BRANCH;1/4;BR            1.0 EA

.2      21306              UNION; BLACK; 1IN; #150           1.0 EA

STANDARD

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

.2      565191             FTG;HOSE;1.00 Hx1 NPT             1.0 EA

; W-1/8 HOLE

..3     14481              FTG;HOSE;1HX1IN NPT;              1.0 EA

(SILVER COLOR)

..3     14481              FTG;HOSE;1HX1IN NPT;              1.0 EA

(SILVER COLOR)

1       100315             UPPER ASY;PCL HINGE               1.0 EA Yes

HL

.2      00944              TAPE;DUCT 3 IN x60YD              1.0 RL

SLVR CLOTH

.2      06197              FTG.;COMP.3/8Tx1/4FPT             2.0 EA

.2      06214              FTG;COMP.1/4Tx1/4MPT 90'          2.0 EA

.2      100096             FACEPLATE/BODY;ASY.;PCL           1.0 EA Yes

..3     03603              COUPLING;HALF;1/4 IN;             2.0 EA

304 SST

..3     03620              COUPLING;HALF;1/2 IN;             2.0 EA

304 SST

..3     100838             CHANNEL; BODY TOP; PCL            2.0 EA

...4    101228             SHEET 10GA(.135) 304L           0.025 EA

SST 2B FIN 48 X 120

..3     100843             PL.;TOP FILLER;PCL                1.0 EA

...4    101228             SHEET 10GA(.135) 304L          0.0074 EA

SST 2B FIN 48 X 120

..3     100906             PIN-RACK GUIDE                    2.0 EA

PCL STEAMER

...4    RB00060            ASMESA479-304-3/8DIA SS         0.104 FT

SMOOTH TURNED BAR

..3     101934             BAR;HNGE CNTER;POLISHED           1.0 EA

PCL-PCS

...4    107007             CENTER HINGE BAR;STM.             1.0 EA

CONV PRO.

..3     107005             BRACKET; TOP HINGE                1.0 EA

STEAM PRO - CONV PRO

...4    RS01305            ASMESA240-304                  0.0069 EA

.240 .015, 48" X 96"

..3     107006             BRACKET;BOTTOM HINGE              1.0 EA

STEAM PRO - CONV PRO

...4    RS01305            ASMESA240-304                  0.0069 EA

.240 .015, 48" X 96"

..3     107008             PLATE;HINGE;CENTER                1.0 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

STEAM PRO-CONV PRO

...4    RS01305            ASMESA240-304                  0.0085 EA

.240 .015, 48" X 96"

..3     20000              STUD;WELD;1/4-20X3/8LG            2.0 EA

CD

..3     40824              WLDMNT; BODY COMP.-PCL            2.0 EA Yes

HL

...4    03628              COUPLING;HALF;3/8IN;304           1.0 EA

SST;MUST BE 9/16 IN LNG

...4    100906             PIN-RACK GUIDE                    2.0 EA

PCL STEAMER

....5   RB00060            ASMESA479-304-3/8DIA SS         0.104 FT

SMOOTH TURNED BAR

...4    100907             PIN-RACK GUIDE-BOT FRT            1.0 EA

PCL STEAMER

...4    69282              SHELL LOWER BODY;PCL              1.0 EA

;HINGED LEFT

....5   101228             SHEET 10GA(.135) 304L           0.182 EA

SST 2B FIN 48 X 120

...4    69283              SHELL UPPER BODY;PCL              1.0 EA

;HINGED LEFT

....5   101228             SHEET 10GA(.135) 304L           0.182 EA

SST 2B FIN 48 X 120

...4    69334              PIN;SHOULDER;0.250DIA x           1.0 EA

0.093

...4    69630              SPACER  STAND-OFF                 6.0 EA

SST (OHIO PART)

...4    70405              TAB BODY WLDMNT;                  2.0 EA

FAB PURPOSES ONLY

....5   RS00430            ASMESA240-304 2B               0.0004 EA

.075in 48 X 120 SS SHEET

..3     42756              DRAIN;MANIFOLD;WLDMNT;            2.0 EA Yes

PCL

...4    03619              COUPLING;HALF;3/8IN;ASME          1.0 EA

SA-105;FRGED STL;BLR RTD

...4    03620              COUPLING;HALF;1/2 IN;             1.0 EA

304 SST

...4    100060             ANGLE;HOLE/DRAIN BODY;            1.0 EA

PCL

....5   101228             SHEET 10GA(.135) 304L        0.003486 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

SST 2B FIN 48 X 120

...4    100061             ANGLE;DRAIN TUBE;PCL              2.0 EA

....5   101228             SHEET 10GA(.135) 304L          0.0006 EA

SST 2B FIN 48 X 120

...4    100062             PL.OUTLET;PCL                     1.0 EA

....5   101228             SHEET 10GA(.135) 304L        0.001494 EA

SST 2B FIN 48 X 120

...4    100063             PL.;SPRAY MTG.;PCL                1.0 EA

....5   101228             SHEET 10GA(.135) 304L        0.001494 EA

SST 2B FIN 48 X 120

...4    100065             ANGLE;DRAIN BODY                  1.0 EA

....5   101228             SHEET 10GA(.135) 304L          0.0035 EA

SST 2B FIN 48 X 120

...4    101925             NIPPLE1.00NPTx 2.00               1.0 EA

SCH 40;SST TOE

..3     52542              BAFFLE; STEAM; J; STMR.           2.0 EA

(VJI)

...4    101469             SHEET 16GA(.060) 304 SST        0.004 EA

; 2B FIN 48 X 120

..3     52873              PLATE;COMPARTMENT BRACE;          2.0 EA

PCL(MAKES 2 FROM PRINT)

...4    RS00710            ASMESA240 304/L 2B              0.036 EA

.135in 48 X 120 SHEET

..3     53125              CHANNEL;SIDE BELLY BAND;          2.0 EA

PCL

...4    RS00710            ASMESA240 304/L 2B              0.333 EA

.135in 48 X 120 SHEET

..3     53763              CHANNEL;COMPT MTG PCL &           1.0 EA

CONV PRO

...4    RS00710            ASMESA240 304/L 2B              0.033 EA

.135in 48 X 120 SHEET

..3     56214              FTG.;COMPARTMENT                 10.0 EA

..3     66658              PLATE REAR PCL 304 SST            1.0 EA

...4    111307             SHEET;10 GA.;(.135) 304        0.1667 EA

SST;2B FINISH;60 x 120

..3     66660              FACEPLATE COMPT PCL 304           1.0 EA

SST

...4    101228             SHEET 10GA(.135) 304L          0.3333 EA

SST 2B FIN 48 X 120

..3     69238              BODY;STEAM SEPERATOR;PCL          2.0 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

STEAMER

...4    00651              SHEET 12GA(.105) 304 SST       0.0666 EA

; 2B FIN 36 X 120 SA-240

..3     69334              PIN;SHOULDER;0.250DIA x          16.0 EA

0.093

..3     69335              PIN- SHOULDER                    16.0 EA

25 DIA X  218 LG

..3     69968              UPPER BODY;STM SEPERATOR          2.0 EA

PCL

...4    00651              SHEET 12GA(.105) 304 SST       0.0057 EA

; 2B FIN 36 X 120 SA-240

.2      101790             INSUL.;REAR WALL;38x29            1.0 EA Yes

PCL/CEM16.

..3     005094000          INSULATION;DUCTWRAP             0.024 RL

1.5 IN; 40 IN x100'

.2      101791             INSUL.COMPT.-BETWEEN;PCL          2.0 EA Yes

10 x 27

..3     005091000          INSUL;DUCT WRAP;1-1.5 IN        0.023 EA

X 10 IN W X 100'

.2      101792             INSUL.,COMPLETE AREA-PCL          2.0 EA Yes

8 1/2 x 13

..3     005092200          INSULATION;DUCTWRAP            0.0103 EA

1.5 IN;22 IN x100'

.2      101793             INSUL.;COMPT.WRAP-PCL             1.0 EA Yes

10FT-3IN x 32 IN

..3     005092200          INSULATION;DUCTWRAP             0.103 EA

1.5 IN;22 IN x100'

.2      102668             LATCH ASY;STM PRO                 2.0 EA Yes

..3     02095              BEARING; FLANGE; .750 ID          2.0 EA

SINTERED BRONZE

..3     100605             SCREW;SOCKET HEAD CAP             2.0 EA       No

;3/8-16X 1.250 LONG;SST

..3     100710             SPRING; CAM ARM RETURN            1.0 EA

..3     1044871            ARM,DOOR LATCH CAM,               1.0 EA

ASSY,PCL

...4    101265             PIN;SPRING LATCH;PCL              1.0 EA

...4    104487             CAM ARM;DOOR LATCH                1.0 EA

PCL/CONV PRO

....5   RS01305            ASMESA240-304                   0.016 EA

.240 .015, 48" X 96"

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

...4    S52532             BUSHING  LATCH (OHIO)             1.0 EA

(SPECIAL ORDER)

....5   RB00240            1 1/2DIA 303SS BAR              0.125 FT

SMOOTH TURNED

..3     14665              NUT;HEX;LOCK;1/4-20;SST;          1.0 EA       No

NYLON INSERT

..3     14675              NUT;HEX;3/8-16;CAD PLTD.          2.0 EA       No

..3     18306              RING;RETAINING;EXTERNAL           1.0 EA

WALDES #5100-75-H

..3     450821             PL,WLDMNT,LATCH,PCL,HL            1.0 EA

...4    100604             BLOCK-ADJ SCREW                   2.0 EA

PCL/PCS

....5   RB00120            3/4DIA  303SS BAR               0.052 FT

C'TRELESS GR(.746-.748)

...4    100605             SCREW;SOCKET HEAD CAP             2.0 EA       No

;3/8-16X 1.250 LONG;SST

...4    101283             STUD;WELD;1/4-20X1/2 IN           1.0 EA

CD SST;NO FLANGE

...4    101405             PIN;SPRING&BUMPER;PCL             1.0 EA

...4    104542             BRACKET;SOLENOID MTG;PCL          1.0 EA

....5   00651              SHEET 12GA(.105) 304 SST       0.0074 EA

; 2B FIN 36 X 120 SA-240

...4    14675              NUT;HEX;3/8-16;CAD PLTD.          2.0 EA       No

...4    66095              PIN- PAWL PIVOT                   1.0 EA

PCL/PCS STEAMER

...4    66650              PL.;LATCH MTG.                    1.0 EA

....5   666501             SHEET 7GA(.180) 304 SSTL          0.1 EA

; 2B FIN 16.375 X 48.00

...4    66673              PIN- LATCH PIVOT                  1.0 EA

PCL/PCS STEAMER

....5   RB00223            1 1/4 DIA -303 S/S ROD          0.218 FT

SMOOTH TURNED

.2      106236             X,HANDLE ASSY,PCL STMR            2.0 EA

..3     1068661            HANDLE;PLASTIC;STEAM/             1.0 EA

CONV-PRO

..3     106908             STUD;HANDLE STEAM/CONV.           1.0 EA

PRO

.2      18309              RING;RETAINING;EXTERNAL           2.0 EA

1/2IN DIA SHAFT

.2      19129              SCREW;HEX;3/8-16x7/8;            12.0 EA       No

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

PLTD;ALLOY STEEL

.2      23101              WASHER;LOCK MEDIUM               18.0 EA       No

PATTRN;3/8IN; ZINC PLTD

.2      23120              WASHER;FLAT;3/8ID;7/8OD          12.0 EA       No

.064/.104 THK;ZINC PLTD

.2      23150              WASHER;LOCK;EXTRNL TOOTH          9.0 EA       No

3/8IN ZNC PLTD

.2      41137              DOOR;ASY.;STEAMPRO                2.0 EA Yes

..3     02096              BEARING;OIL IMPREGNATED           1.0 EA

.753 ID;POROUS BRONZE

..3     100546             DOOR WELDMT  PCL REVE             1.0 EA

..3     100557             SCREW;SOCKET HD;SST;              2.0 EA       No

1/4-28x1

..3     101788             SHIM;DR;PCL;16GA.                 2.0 EA

...4    RS00341            ASMESA240-304 .060in 2B         0.009 EA

48 X 120 SS SHEET

..3     101818             COVER ASY;DR-CONVPRO;PCL          1.0 EA Yes

...4    100759             BAR;DR HANDLE;PCL                 1.0 EA

....5   RB00691            1" +0.040"BLK EXTR.ACRYL          1.3 FT

BAR PROTECTED GLOSS FINI

...4    100990             SPRING; WAVE; PCL                 2.0 EA

...4    101044             BRACKET;DOOR HANDLE               2.0 EA

...4    101048             PAD; INSULATOR; PLASTIC           2.0 EA

(DOOR BRACKET)

...4    19288              SCREW;1/4-20x3/4;HEX HD;          4.0 EA       No

ZINC PLATED; GRADE 8

...4    23105              WASHER; LOCK; MED PATT            4.0 EA       No

1/4IN KATLINK STYLE; SST

...4    23116              WASHER;FLAT;.281 IDx.625          4.0 EA       No

OD. x .065 THK;SST

...4    54230              WLDMNT;COVER DOOR PCL             1.0 EA

STEAMER

....5   112659             COVER;DOOR;PCL STEAMER            1.0 EA

FORMED

.....6  RS00476            ASMESA240-304 #4 SPV-1           0.25 EA

.075in 48 X 120 SS SHEET

..3     104292             GASKET;DOOR-PCL CUT 84            1.0 EA

IN; ENDS GLUED TOGETHER

..3     107507             ADHESIVE;LOCTITE;BLUE;            0.0 EA       No

THRDLKER 242;50ML;#24231

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

..3     109420             SCREW;SET;CUP POINT; HEX          1.0 EA       No

SOCKET;1/4-20 X .375;SST

..3     109823             PIN;ROLLER;PCL/CONV PRO           1.0 EA

..3     109826             RING;RETAINING;SPIRAL;            1.0 EA

.750 DIA. SHAFT;SST

..3     19123              SCREW;SOCKET HD;                  4.0 EA       No

1/4-28X1.5;SST

..3     23098              WASHER;.75IDx1.1880Dx.05          3.0 EA       No

S/S

..3     52061              BAR;HINGE PIVOT                   1.0 EA

..3     68084              ROLLER- DOOR LATCH                1.0 EA

PCL/PCS STEAMER

...4    S68084             ROLLER- DOOR LATCH                1.0 EA

PCL/PCS STEAMER

....5   RB00240            1 1/2DIA 303SS BAR              0.066 FT

SMOOTH TURNED

..3     WR50193            PIN;SPRING;1/4 IN X 1IN           1.0 EA       No

LG;SST

..3     WR50193            PIN;SPRING;1/4 IN X 1IN           1.0 EA       No

LG;SST

.2      441481             PAWL;WLDMNT;PCL;H.L.              2.0 EA

..3     100855             PAWL;BAR;PCL                      1.0 EA

...4    00424              BAR;.25X1;304;SST               0.469 FT

...4    RB00730            1/4X 1-304-HRAP A-276-            0.5 FT

STAINLESS STEEL BAR

..3     100856             PAWL;ANGLE;PCL                    1.0 EA

...4    00403              ANGLE;1/8X1X1;304SST            0.083 FT

..3     52540              BUSHING;PAWL PIVOT;               1.0 EA

STEAMPRO/CONVPRO

..3     66079              LOCK;PAWL LATCH                   1.0 EA

...4    RS01305            ASMESA240-304                   0.014 EA

.240 .015, 48" X 96"

.2      45099              DOOR HARDWARE                     1.0 EA Yes

..3     02093              BEARING -.566 ID .753 OD          4.0 EA

POROUS BRONZE

..3     100004             BAR;NUT;(3)3/8-16 HOLES;          3.0 EA

PCL

..3     100539             WASHER; DOOR HINGE                4.0 EA       No

..3     101231             SCREW; 10-24 X 0.500 SST          4.0 EA       No

PHILLIPS TRUSS HEAD

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

..3     101935             BAR;HINGE BOTTOM                  1.0 EA

...4    100001             BAR- HINGE BOTTOM                 1.0 EA

(HINGED LEFT)

....5   RB00790            1/2X 1 1/2-304-HRAP               0.7 FT

A276 S S  FLAT TRUE BAR

...4    RB00790            1/2X 1 1/2-304-HRAP               0.6 FT

A276 S S  FLAT TRUE BAR

..3     101936             BAR;HINGE TOP                     1.0 EA

...4    100000             BAR- HINGE TOP                    1.0 EA

(HINGED LEFT)

....5   RB00790            1/2X 1 1/2-304-HRAP               0.7 FT

A276 S S  FLAT TRUE BAR

...4    RB00790            1/2X 1 1/2-304-HRAP               0.6 FT

A276 S S  FLAT TRUE BAR

..3     19134              SCREW;3/8-16x1;HH;SST             6.0 EA       No

..3     53124              CHANNEL;DOOR END;PCL              2.0 EA

POLISHED;J STMR

...4    RS00476            ASMESA240-304 #4 SPV-1         0.0294 EA

.075in 48 X 120 SS SHEET

1       100768             CYLINDER;PRESSURE LOCK            2.0 EA

1       100771             SOLENOID;CONT DUTY;CLASS          2.0 EA

F; INSUL;120VAC;60HZ

1       100883             FTG.;COMPRESSION;TEE              2.0 EA

3/8Tx1/4NPTx3/8T;BRASS

1       100914             BRACKET;CYLINDER MNTG             2.0 EA

.2      00651              SHEET 12GA(.105) 304 SST        0.001 EA

; 2B FIN 36 X 120 SA-240

1       101012             PIN;SOLENOID;PCL;.125             2.0 EA

DIA' AISI TYPE 304 SST

1       101013             STRAP;SOLENOID;PCL                2.0 EA

.2      101013F            STRAP;SOLENOID;SHEARED            1.0 EA

..3     101469             SHEET 16GA(.060) 304 SST     0.000518 EA

; 2B FIN 48 X 120

1       101015             SPRING;LATCH;PCL                  2.0 EA

1       101061             MANIF;WATER;ASY;PCL               1.0 EA Yes

.2      00934              SEALANT; PIPE DOPE;ONE            0.0 EA

PINT CAN,RECTORSEAL #5

.2      03050              CAP;PIPE;ROUND;1/2 IN;            1.0 EA

BRASS

.2      06214              FTG;COMP.1/4Tx1/4MPT 90'          4.0 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

.2      100976             MANIF.WATER;VALVE SPPT            1.0 EA

;PCL

..3     RP00320            1/2SCH 80 RED BRASS              1.45 FT

PIPE

.2      14304              NIPPLE .25NPTxCLOSE SCH           2.0 EA

40; BRASS TBE

.2      14411              NIPPLE .25NPTx 1.50; SCH          2.0 EA

40; BRASS TBE

.2      21303              UNION; 1/2IN; BRASS               1.0 EA

.2      22236              VALVE; SOLENOID; 3 WAY.;          2.0 EA

60 HZ.

.2      22241              VLVE;SOLENOID-2 WAY NC 1          2.0 EA

T; ORIFICE 5/32 120V/60

1       101062             COMPARTMENT;CONDENSER AS          2.0 EA Yes

Y(PCL)

.2      02549              BUSHING;CONDENSER;CONV.           1.0 EA

STMR.1/2"X1/4X1/8

..3     RB01210            7/8HEX YELLOW BRASS BAR         0.078 FT

.2      02594              BUSHING;HEX;3/8x1/4;BR            1.0 EA

.2      05235              ELBOW;STREET;90 DEG.;             1.0 EA

1/4 IN; BR.;LONG PATTERN

.2      05244              ELBOW;90 DEG.;3/8 IN;BR           1.0 EA

.2      06193              FTG.;COMP.1/4Tx3/8MPT             1.0 EA

.2      100109             NOZZLE;SPRAY                      1.0 EA

.2      100982             VALVE;PRESSURE RELIEF             1.0 EA

.2      14304              NIPPLE .25NPTxCLOSE SCH           1.0 EA

40; BRASS TBE

1       101794             VALVE;STEAM;ASY;PCL               2.0 EA Yes

.2      00934              SEALANT; PIPE DOPE;ONE            0.0 EA

PINT CAN,RECTORSEAL #5

.2      02566              BUSHING;HEX;3/4x1/2;BR            1.0 EA

.2      05231              ELBOW;STREET;3/4x90;BR            1.0 EA

FORGED STYLE ONLY

.2      06240              FTG;HOSE BARB;3/4x3/4MPT          1.0 EA

BR.

.2      102362             ELBOW;UNION;MALE TO FEM           1.0 EA

1/2IN;BRASS

.2      14324              NIPPLE .50NPTxCLOSE; SCH          1.0 EA

40; BRASS TBE

.2      22193              VALVE; SOLENOID 3/4X3/4;          1.0 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

120V; 60 HZ

1       102074             TUBING;Cu-3/8X21.25LG             2.0 EA Yes

-FORMED

.2      00729              TUBE;Cu;SEAMLESS; .375          2.563 FT

x.032WALL 10'SOFT STR

1       102077             TUBING-FORMED (LO COMP-           2.0 EA Yes

STM TR/DRAIN FIT)-PCL

.2      00728              TUBING;Cu; SEAMLESS.25          0.583 FT

ODX.028W-10'SOFT STR

1       102078             TUBE;FORMED;PINCH VALVE           2.0 EA Yes

/DRAIN;PCL

.2      00728              TUBING;Cu; SEAMLESS.25          0.708 FT

ODX.028W-10'SOFT STR

1       102079             TUBING-FORMED UP COMP-            1.0 EA Yes

VALVE/DRAIN -PCL

.2      00728              TUBING;Cu; SEAMLESS.25          1.167 FT

ODX.028W-10'SOFT STR

1       102080             TUBING-FORMED (COMPART-           2.0 EA Yes

MENT/STM TRAP) - PCL

.2      00728              TUBING;Cu; SEAMLESS.25          1.792 FT

ODX.028W-10'SOFT STR

1       102081             TUBING-FORMED LO COMP-            1.0 EA Yes

VALVE/DRAIN-PCL

.2      00728              TUBING;Cu; SEAMLESS.25          0.875 FT

ODX.028W-10'SOFT STR

1       102082             TUBING-FORMED (UP COMP-           1.0 EA Yes

VALVE/FL RESTRICTOR)-PCL

.2      00728              TUBING;Cu; SEAMLESS.25          1.396 FT

ODX.028W-10'SOFT STR

1       102082             TUBING-FORMED (UP COMP-           1.0 EA Yes

VALVE/FL RESTRICTOR)-PCL

.2      00728              TUBING;Cu; SEAMLESS.25          1.396 FT

ODX.028W-10'SOFT STR

1       102083             TUBING-FORMED LO COMP-            1.0 EA Yes

VALVE/FL RESTOR - PCL

.2      00728              TUBING;Cu; SEAMLESS.25          1.833 FT

ODX.028W-10'SOFT STR

1       102085             TUBING-FORMED CONDENSER           2.0 EA Yes

BOX/VALVE - PCL

.2      00728              TUBING;Cu; SEAMLESS.25          1.833 FT

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

ODX.028W-10'SOFT STR

1       104452             TUBING;COPPER SEAMLESS            1.0 EA Yes

.375X17LG.

.2      00729              TUBE;Cu;SEAMLESS; .375          1.417 FT

x.032WALL 10'SOFT STR

1       104453             TUBING;Cu SEAMLESS                1.0 EA Yes

.375X.032WALLX11.75LG

.2      00729              TUBE;Cu;SEAMLESS; .375          0.979 FT

x.032WALL 10'SOFT STR

1       14618              NUT;HEX;1/4-20;300 SST;           8.0 EA       No

FULL FINISHED

1       14661              NUT;COMP. FTG;1/4 TUBE            4.0 EA

;BRASS

1       14674              NUT;HEX;6-32;ZINC PLTD           12.0 EA       No

1       146771             NUT; ACORN; 1/4-20; SST;          2.0 EA       No

THREADLOCK (LOW CROWN)

1       18313              O-RING;005 TRADE SIZE;            4.0 EA

EP;7/64IDx15/64ODx1/16W

1       19259              SCREW,RD HD, 6-32X1/2             8.0 EA

1       19288              SCREW;1/4-20x3/4;HEX HD;          8.0 EA       No

ZINC PLATED; GRADE 8

1       20559              TRAP;THERMOSTATIC 1/4IN           2.0 EA

COMP X 1/4IN COMP

1       23105              WASHER; LOCK; MED PATT            8.0 EA       No

1/4IN KATLINK STYLE; SST

1       23111              WASHER; LOCK; INTERNAL           12.0 EA       No

TOOTH;#6; ZINC PLTD

1       23116              WASHER;FLAT;.281 IDx.625         16.0 EA       No

OD. x .065 THK;SST

1       45093              RACK,STEAM TUBE,MTG.HDW.          1.0 EA

1       69298              SCREEN;DRAIN COMPARTMENT          2.0 EA

(STAINLESS)

.2      RS00341            ASMESA240-304 .060in 2B        0.0048 EA

48 X 120 SS SHEET