
[image: image4.wmf] 



Only Cleveland Range authorized technicians should perform the installation of the Cleveland Range Heat Shield. Read entire installation procedure before beginning.  These instructions are for Heat Shield installation on models 24CGA10, 24CEA10, and 24CDP10.
Field Kit (P/N 113716) PARTS LIST
	ITEM 
	QUANTITY
	PART NUMBER
	DESCRIPTION

	1
	1
	113295
	Heat Shield

	2
	1
	108860
	Left Hanger Bracket

	3
	1
	108861
	Right Hanger Bracket

	4
	2
	108644
	1/4-20 x 1/2 Carriage Screw

	5
	4
	111575
	1/4-20 x 5/8 Carriage Screw

	6
	6
	14665
	1/4-20 Lock Nut

	7
	6
	C8005050
	Washer

	8
	6
	02616
	Snap-In Bushing

	9
	2
	111907
	10-32 x 3/4 Socket Head Cap Screw

	10
	2
	14672
	10-32 Lock Nut

	11
	2
	101655
	#10 Washer

	12
	1
	260BHL
	Instructions


Tool List

1ea
“Sharpie” marker

1ea
drill

1ea
Phillips Screwdriver
1ea
3/16” drill bit

1ea
Allen wrench
1ea
3/8” nut driver

1ea
7/16” nut driver


[image: image1.jpg]...........
.....

¥
AR




Figure 1
1. Turn off the oven and make sure the oven is cool enough to work around it. 

2. Unplug or disconnect the power supply from the oven. Turn off the water supply for the oven. 
3. Remove the protective paper from the Heat Shield and Hanger Brackets. Attach Hanger Brackets onto Heat Shield.  Use the 1/4 -20 Carriage Screws and Lock Nuts.  See Figure 2.
4. Insert Bushings into Heat Shield.  See Figure 2.
5. Plug remaining holes with Carriage Screws, Lock Nuts, and Washers.  See Figure 2.

6. Place Heat Shield onto Steamer with Hanger Brackets placed on the top sheeting.  The Heat Shield should be against the side sheeting and flush with front face of the steamer.

7. Mark the location of the Hanger Bracket holes on the top sheeting. Remove the Heat Shield.

8. Remove the side sheeting from the side of the unit the Heat Shield is to be installed.
9. Drill 3/6” diameter hole at the marked locations.

10. Install the Socket Head Cap Screw, washers and locknuts.

11. Re-install the side sheeting.

12. Install the Heat Shield with the Hanger Brackets located on the Socket Head Cap Screw.  See Figure 3.
[image: image2.jpg]



Figure 2
[image: image3.jpg]3 e
Rol oo RN S




Figure 3

Cleveland Range, LLC

1333 East 179th St, Cleveland, Ohio 44110

Drawing No. 260BHL
Revision Index Page

	LET
	REVISION HISTORY
	DATE
	BY

	A
	PRODUCTION RELEASE PER ECR-11-268
	04/27/12
	RCB

	
	
	
	

	
	
	
	


Installation Instructions


Field Installation for 10 Pan Steamer Heat Shield








1333 East 179th Street


Cleveland, Ohio 44110





Phone: 216-481-4900


Fax: 216-481-3782


www.clevelandrange.com








� EMBED Word.Picture.8  ���





Attach Right Hanger Bracket








Insert Bushings








Plug Holes





Hanger Bracket located on the Socket Head Cap Screw





This Page for Internal Record Only, Not for Printing








3

_1314688944.doc
[image: image1.emf]





