Parent Item/BOM Code: PCSM00003            36PCSM,MCS                    EA

As Of: 04/30/20  Levels:                Rev:

PCO Number:               ID:             Domain:          Output: page

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

Parent  PCSM00003          36PCSM,MCS                            EA

1       101527             PNL;SIDE;430SST;STD;              2.0 EA Yes

ASSY;LOUVERED

.2      101486             CLIP; BASE SIDE PNL; SST          2.0 EA

..3     RS00341            ASMESA240-304 .060in 2B        0.0009 EA

48 X 120 SS SHEET

.2      18358              RIVET;FLUSH BREAK;1/8IN           4.0 EA       No

DIA;.126-.187 GRIP; SST

.2      66012              PNL;SIDE;FULL;430;                1.0 EA

LOUVERED;SST;BASE

..3     RS00187            .036in, 430 #4 BLUE-1          0.1667 EA

48 X 120 SS SHEET

1       101690A            BASE ASY.;36PCSM                  1.0 EA

.2      007281425          TUBE;Cu;.25 ODx14.25              1.0 EA Yes

..3     00728              TUBING;Cu; SEAMLESS.25          1.188 FT

ODX.028W-10'SOFT STR

.2      00934              SEALANT; PIPE DOPE;ONE            0.0 EA

PINT CAN,RECTORSEAL #5

.2      03197              CLAMP;1 HOLE;RIGID;3/4IN          1.0 EA

.2      03204              CLAMP;WORM DRIVE;.812-            2.0 EA

1.50 DIA.

.2      03204              CLAMP;WORM DRIVE;.812-            2.0 EA

1.50 DIA.

.2      06217              FTG;COMP.3/8Tx3/8MPT 90'          1.0 EA

.2      085110800          HOSE;WHT;EPDM;3/4IDx8.0           1.0 EA Yes

0LG

..3     08511              HOSE;WHITE EPDM;3/4 IN          0.667 FT

ID;FDA APPROVED,150 PSI

.2      085111100          HOSE;EPDM; 3/4 ID x 11            1.0 EA Yes

..3     08511              HOSE;WHITE EPDM;3/4 IN          0.917 FT

ID;FDA APPROVED,150 PSI

.2      101231             SCREW; 10-24 X 0.500 SST          1.0 EA       No

PHILLIPS TRUSS HEAD

.2      1017031            PIPING ASY RETURN;                1.0 EA Yes

STM COIL LARGE;600MM

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

..3     00934              SEALANT; PIPE DOPE;ONE            0.0 EA

PINT CAN,RECTORSEAL #5

..3     05259              ELBOW;STREET;90 DEG.;             1.0 EA

3/4 IN; BLK.

..3     05270              ELBOW;90 DEG.;1-1/4x3/4           1.0 EA

BLK.;REDUCING

..3     14344              NIPPLE .75NPTx 2.00; SCH          2.0 EA

40; BLK IRON TBE

..3     14354              NIPPLE .75NPTx 5.00; SCH          1.0 EA

40; BLK IRON TBE

..3     16602              PLUG;SQ HD;1/2IN PIPE;            1.0 EA

BLK

..3     19872              STRAINER;Y PIPE FITTING;          1.0 EA

3/4NPT;CAST IRON BODY

..3     20555              TRAP; BUCKET #60 3/4IN            1.0 EA

80#

..3     21305              UNION;PIPE;3/4IN;BLK.;            1.0 EA

150#; NO WARRICKS

.2      101723             STEAM INLET ASY.PCL               1.0 EA Yes

STEAM COIL

..3     00934              SEALANT; PIPE DOPE;ONE            0.0 EA

PINT CAN,RECTORSEAL #5

..3     02557              BUSHING;HEX;1/2x1/4 BLK           1.0 EA

..3     02598              BUSHING;REDUCING;1x3/4;           2.0 EA

HEX;BLACK

..3     03277              VALVE;BALL; 3/4 IN                1.0 EA

MALE/FEM

..3     03665              COUPLING;FULL;1/4 IN;304          1.0 EA

SST

..3     05259              ELBOW;STREET;90 DEG.;             1.0 EA

3/4 IN; BLK.

..3     05270              ELBOW;90 DEG.;1-1/4x3/4           1.0 EA

BLK.;REDUCING

..3     07169              GAUGE;BOT MOUNT;0-100PSI          1.0 EA

2-1/2INx1/4IN;BR.MVMNT

..3     101699             VALVE;SOLENOID;STEAM              1.0 EA

STEAMCOIL 1 IN PIPE;120V

..3     14343              NIPPLE .75NPTxCLOSE; SCH          1.0 EA

40; BLK IRON TBE

..3     14344              NIPPLE .75NPTx 2.00; SCH          2.0 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

40; BLK IRON TBE

..3     14354              NIPPLE .75NPTx 5.00; SCH          1.0 EA

40; BLK IRON TBE

..3     16820              SYPHON;PIGTAIL 1/4IN IPS          1.0 EA

FIG. #126

..3     20205              TEE; 3/4IN; BLACK                 1.0 EA

..3     20209              TEE; 3/4IN X 3/4IN X              1.0 EA

1/2IN; BLACK

..3     21305              UNION;PIPE;3/4IN;BLK.;            1.0 EA

150#; NO WARRICKS

..3     KE54941-2          SAFETY VLV;50#,3/4"X3/4"          1.0 EA

CONBRACO #10-322-P50T

.2      101873             NUT;HEX;LOCK;THIN;1/4-20          2.0 EA       No

;PLTD;NYLON INSERT

.2      1041093900         WIRE;Cu;18GAx39.00 LG             1.0 EA Yes

;BLACK

..3     104109             WIRE; Cu-AWM-18-16/30            3.25 FT

105 C; BLACK

.2      1041113700         WIRE;Cu;18GAx37.00 LG             1.0 EA Yes

;BROWN

..3     104111             WIRE; Cu-AWM-18-16/30          3.0833 FT

105 C; BROWN

.2      1041153900         WIRE;Cu;18GAx39.00 LG             1.0 EA Yes

;PINK

..3     104115             WIRE; Cu-AWM-18-16/30            3.25 FT

105 C; PINK

.2      1041163500         WIRE;Cu;18GAx35.00 LG             2.0 EA Yes

;PURPLE

..3     104116             WIRE; Cu-AWM-18-16/30          2.9167 FT

105 C; PURPLE

.2      1041173900         WIRE;Cu;18GAx39.00 LG             1.0 EA Yes

;RED

..3     104117             WIRE; Cu-AWM-18-16/30            3.25 FT

105 C; RED

.2      105245             CONNECTOR;GRNFLD;3/8 IN           1.0 EA

X 90 DEG;SNAPTITE STYLE

.2      105247             CONNECTOR;GREENFIELD;1/2          1.0 EA

90D;SNAPTITE STYLE

.2      1052480750         CONDUIT;GREENFIELD; 3/8           1.0 EA Yes

x7.5 LG

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

..3     105248             CONDUIT;ELEC;GREENFIELD         0.625 FT

;3/8 IN;FLEXIBLE

.2      1052481300         CONDUIT;GREENFIELD; 3/8           1.0 EA Yes

x13 LG

..3     105248             CONDUIT;ELEC;GREENFIELD         1.083 FT

;3/8 IN;FLEXIBLE

.2      1052481600         CONDUIT;GREENFIELD; 3/8           1.0 EA Yes

x16 LG

..3     105248             CONDUIT;ELEC;GREENFIELD         1.333 FT

;3/8 IN;FLEXIBLE

.2      1052481700         CONDUIT;GREENFIELD;3/8            1.0 EA Yes

17 LG

..3     105248             CONDUIT;ELEC;GREENFIELD        1.1416 FT

;3/8 IN;FLEXIBLE

.2      1052491800         CONDUIT;GREENFIELD;1/2            1.0 EA Yes

18.00 LG

..3     105249             CONDUIT;ELEC;GREENFIELD           1.5 FT

1/2IN;FLEXIBLE

.2      105560             BUSHING;ANTISHORT;1/2 IN          2.0 EA

TRADE SIZE #3

.2      105561             BUSHING;ANTISHORT;3/8 IN          2.0 EA

TRADE SIZE #1

.2      105561             BUSHING;ANTISHORT;3/8 IN          2.0 EA

TRADE SIZE #1

.2      105561             BUSHING;ANTISHORT;3/8 IN          2.0 EA

TRADE SIZE #1

.2      105561             BUSHING;ANTISHORT;3/8 IN          2.0 EA

TRADE SIZE #1

.2      1058501500         HOSE;1/4 IDx15;SILICONE           1.0 EA Yes

REINFORCED

..3     105850             HOSE;POLY REINFORCED;            1.25 FT

1/4 IN IDx1/2IN OD

.2      105970             BOLT;U;1/4-20;3/4 IN              1.0 EA       No

PIPE

.2      106527             CONNECTOR;GRNFLD;3/8X             1.0 EA

90DEG.; LOCKNUT STYLE

.2      1073121            CLAMP;HOSE;1/4 ID;METAL           2.0 EA       No

TENSION BAND

.2      109232             SCREW;1/4-20x1/2;HEX HD;          2.0 EA       No

SST

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

.2      111275             X,COVER,ELEC BOX,STEAM            1.0 EA

COIL,LARGE

..3     RS01425            .060in, ALUMINIZED STEEL        0.021 EA

48 X 120 SHEET

.2      111276             BRACKET;PIPING ASY;               1.0 EA

STEAM SUPPLY;LG STM COIL

..3     101228             SHEET 10GA(.135) 304L          0.0014 EA

SST 2B FIN 48 X 120

.2      111277             MANUAL BLOWDOWN ASY;              1.0 EA Yes

STEAM COIL LARGE

..3     00934              SEALANT; PIPE DOPE;ONE            0.0 EA

PINT CAN,RECTORSEAL #5

..3     03277              VALVE;BALL; 3/4 IN                1.0 EA

MALE/FEM

..3     05231              ELBOW;STREET;3/4x90;BR            1.0 EA

FORGED STYLE ONLY

..3     06240              FTG;HOSE BARB;3/4x3/4MPT          1.0 EA

BR.

..3     14342              NIPPLE .75NPTxCLOSE; SCH          1.0 EA

40; BRASS TBE

..3     16607              PLUG;SQ HD;3/4;BRASS              1.0 EA

..3     20206              TEE; 3/4IN; BRASS                 1.0 EA

.2      111292             BOILER PARTS LIST;ROUND           1.0 EA Yes

STEAM COIL

..3     00934              SEALANT; PIPE DOPE;ONE            0.0 EA

PINT CAN,RECTORSEAL #5

..3     03276              VALVE;BALL 1/4 IN BRASS;          1.0 EA

MALE/FEMALE; FULL PORT

..3     05258              ELBOW;90 DEG.;3/4 IN;BLK          1.0 EA

..3     07108              GASKET;WATER GAUGE;FIBER          2.0 EA

11/16x5/8x1/32

..3     07176              1/2GAUGE;SET WATER(COMP)          1.0 EA

2 VALVES w/washrs gaskts

..3     07302              X,GAUGE,WATER,5/8"x6.00           1.0 EA

PYREX RED LINE; GLASS

..3     101112             BOX;PROBE COVER;LARGE             1.0 EA

STEAM COIL

...4    100781             SHEET 16GA(.060) T140          0.0111 EA

COATED STEEL 48 X 120

..3     101912             EXTENSION;PROBE;LOW WATR          1.0 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

PCL STEAM COIL

..3     101913             EXTENSION;PROBE;HIGH              1.0 EA

WATR;PCL STEAM COIL

..3     14343              NIPPLE .75NPTxCLOSE; SCH          1.0 EA

40; BLK IRON TBE

..3     14618              NUT;HEX;1/4-20;300 SST;           2.0 EA       No

FULL FINISHED

..3     16671              PROBE;LIQUID LEVEL;               2.0 EA

4-5/16 LG(316 S/S)

..3     22131              VALVE;SAFETY;3/4IN;15PSI          1.0 EA

BRZ;SST SPRING;SIDE OUT

..3     23105              WASHER; LOCK; MED PATT            2.0 EA       No

1/4IN KATLINK STYLE; SST

..3     23116              WASHER;FLAT;.281 IDx.625          2.0 EA       No

OD. x .065 THK;SST

..3     260AXL             INSTRCTION;ASSY;STM COIL          0.0 EA

;LARGE

.2      1112951            TUBE;3/8 OD;GEN TO GAUGE          1.0 EA Yes

ROUND STM COIL;36 CONV

..3     00729              TUBE;Cu;SEAMLESS; .375         2.2396 FT

x.032WALL 10'SOFT STR

.2      1112965            TUBE;3/8OD;MANIF/                 1.0 EA Yes

HI LIMIT PR. SW.;36PCSM

..3     00729              TUBE;Cu;SEAMLESS; .375          1.906 FT

x.032WALL 10'SOFT STR

.2      1112975            TUBE;3/8 OD;MANIF/OP PR.          1.0 EA Yes

SWITCH;36PCSM

..3     00729              TUBE;Cu;SEAMLESS; .375          1.821 FT

x.032WALL 10'SOFT STR

.2      111303             AUTO BLOWDOWN ASY;36CSM           1.0 EA Yes

LARGE STEAM COIL

..3     00934              SEALANT; PIPE DOPE;ONE            0.0 EA

PINT CAN,RECTORSEAL #5

..3     02623              BUSHING;HEX;1/2x1/4;BR            2.0 EA

..3     05235              ELBOW;STREET;90 DEG.;             1.0 EA

1/4 IN; BR.;LONG PATTERN

..3     05295              ELBOW;UNION;FEM TO FEM            1.0 EA

1/2IN X 1/2IN;BR.

..3     06241              FTG;HOSE BARB;3/4Hx               1.0 EA

1/2MPT;BR.

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

..3     105245             CONNECTOR;GRNFLD;3/8 IN           1.0 EA

X 90 DEG;SNAPTITE STYLE

..3     109866             FTG;HOSE BARB;RADIUS TIP          1.0 EA

1/4 ID HOSE

..3     14304              NIPPLE .25NPTxCLOSE SCH           1.0 EA

40; BRASS TBE

..3     14328              NIPPLE .50NPTx 2.00; SCH          1.0 EA

40; BRASS TBE

..3     14331              NIPPLE .50NPTx 2.50; SCH          1.0 EA

40; BRASS TBE

..3     222121             VALVE;BALL;1/2IN;FEM/FEM          1.0 EA

W/BUILT IN CHECK VALVE

..3     222213             VALVE;SOL;72IN LEADS;             1.0 EA

1/2IN DRAIN,120vAC,60hz

.2      111364             ASY;DRAIN MANIF;36CSM16           1.0 EA Yes

..3     00934              SEALANT; PIPE DOPE;ONE            0.0 EA

PINT CAN,RECTORSEAL #5

..3     03616              COUPLING;1-1/2x3/4;BLACK          1.0 EA

REDUCING

..3     05237              ELBOW;STREET;45 DEG.;             1.0 EA

3/4 IN;BLK.

..3     05259              ELBOW;STREET;90 DEG.;             2.0 EA

3/4 IN; BLK.

..3     05292              ELBOW;STREET;90DEG.               1.0 EA

1-1/2 IN; BLK

..3     06192              FTG.;COMP.1/4Tx1/8MPT 90          1.0 EA

DEG

..3     06240              FTG;HOSE BARB;3/4x3/4MPT          2.0 EA

BR.

..3     06241              FTG;HOSE BARB;3/4Hx               1.0 EA

1/2MPT;BR.

..3     106296             BUSHING;REDUCER;1-1/2             1.0 EA

x1/2;BLACK

..3     110531             ELBOW;SIDE OUTLET;1.500;          1.0 EA

BLACK IRON

..3     132521             MANIFOLD;DRAIN;w/ STUD            1.0 EA Yes

...4    13252              X,MANIFOLD,1.5"x8.5"LG            1.0 EA

(FOR SRVC USE #132521)

...4    19917              STUD;ARC WELD;1/4-20 UNC          1.0 EA

2Ax.875 SST ASME SA-479;

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

..3     14343              NIPPLE .75NPTxCLOSE; SCH          1.0 EA

40; BLK IRON TBE

..3     14481              FTG;HOSE;1HX1IN NPT;              1.0 EA

(SILVER COLOR)

..3     22211              VALVE;SWING CHECK;3/4IN           1.0 EA

W/TEFLON DISC-STEAM USE

..3     565191             FTG;HOSE;1.00 Hx1 NPT             1.0 EA

; W-1/8 HOLE

...4    14481              FTG;HOSE;1HX1IN NPT;              1.0 EA

(SILVER COLOR)

...4    14481              FTG;HOSE;1HX1IN NPT;              1.0 EA

(SILVER COLOR)

.2      1113651            PIPING STM OUTLET;36PCSM          1.0 EA Yes

;LARGE STEAM COIL

..3     00934              SEALANT; PIPE DOPE;ONE            0.0 EA

PINT CAN,RECTORSEAL #5

..3     02563              BUSHING;HEX;3/4x1/4;BR            2.0 EA

..3     02570              BUSHING;HEX;1x3/4 IN              1.0 EA

BRASS

..3     03277              VALVE;BALL; 3/4 IN                1.0 EA

MALE/FEM

..3     03732              CROSS;4-WAY;3/4 IN;BR.            1.0 EA

..3     05231              ELBOW;STREET;3/4x90;BR            1.0 EA

FORGED STYLE ONLY

..3     06240              FTG;HOSE BARB;3/4x3/4MPT          2.0 EA

BR.

..3     100660             TEE;3/4 SIDE OUTLET;BR            1.0 EA

..3     100883             FTG.;COMPRESSION;TEE              1.0 EA

3/8Tx1/4NPTx3/8T;BRASS

..3     101207             TRAP;THRMO W/POL.SEAT             1.0 EA

1/4MPTX1/4 IN COMP

..3     14342              NIPPLE .75NPTxCLOSE; SCH          3.0 EA

40; BRASS TBE

..3     14346              NIPPLE .75NPTx 2.50; SCH          1.0 EA

40; BRASS TBE

..3     16607              PLUG;SQ HD;3/4;BRASS              3.0 EA

..3     20206              TEE; 3/4IN; BRASS                 2.0 EA

.2      1113661            ASY;WATER FEED;36PCSM;            1.0 EA Yes

LGE STM COIL

..3     00934              SEALANT; PIPE DOPE;ONE            0.0 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

PINT CAN,RECTORSEAL #5

..3     02594              BUSHING;HEX;3/8x1/4;BR            1.0 EA

..3     05235              ELBOW;STREET;90 DEG.;             1.0 EA

1/4 IN; BR.;LONG PATTERN

..3     05236              ELBOW;90 DEG.;1/4 IN BR           1.0 EA

..3     05244              ELBOW;90 DEG.;3/8 IN;BR           2.0 EA

..3     05280              ELBOW;STREET;3/8 IN;BR            1.0 EA

..3     07169              GAUGE;BOT MOUNT;0-100PSI          1.0 EA

2-1/2INx1/4IN;BR.MVMNT

..3     100064             CLAMP;MOUNT FOR 1/4 TEE/          1.0 EA

ELBOW;TUBE FITTING

...4    RS00430            ASMESA240-304 2B                0.001 EA

.075in 48 X 120 SS SHEET

..3     100303             CLAMP;MOUNT;3/8 TEE &             1.0 EA

ELBOW & TUBE FITTING

...4    RS00430            ASMESA240-304 2B               0.0015 EA

.075in 48 X 120 SS SHEET

..3     100339             STRAINER;#27-40 MESH;             1.0 EA

3/8 IN NPT

..3     100884             PLATE;MTG.;2 WATER                1.0 EA

INLETS

...4    RS00430            ASMESA240-304 2B               0.0027 EA

.075in 48 X 120 SS SHEET

..3     105245             CONNECTOR;GRNFLD;3/8 IN           1.0 EA

X 90 DEG;SNAPTITE STYLE

..3     105277             FTG;HOSE BARB;3/8HX1/4            1.0 EA

NPT;BR

..3     111289             FTG;HOSE BARB;90 DEG;1/4          1.0 EA

H x 1/4NPT MALE

..3     14304              NIPPLE .25NPTxCLOSE SCH           1.0 EA

40; BRASS TBE

..3     14310              NIPPLE .25NPTx 4.00 SCH           1.0 EA

40; BRASS TBE

..3     14411              NIPPLE .25NPTx 1.50; SCH          1.0 EA

40; BRASS TBE

..3     14427              NIPPLE .38NPTxCLOSE; SCH          2.0 EA

40; BRASS TBE

..3     14442              NIPPLE .38NPTx 1.50; SCH          1.0 EA

40; BRASS TBE

..3     14618              NUT;HEX;1/4-20;300 SST;           4.0 EA       No

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

FULL FINISHED

..3     19288              SCREW;1/4-20x3/4;HEX HD;          4.0 EA       No

ZINC PLATED; GRADE 8

..3     19870              STRAINER;1/4IN WATTS V            1.0 EA

#27; 40 MESH SCREEN

..3     20199              TEE;1/4IN;BRASS                   1.0 EA

..3     22223              VALVE;SOLENOID;1/4NPT;            1.0 EA

36IN LEADS

..3     22239              VALVE; PRESSURE RED;              1.0 EA

R; 3/8NPT

..3     23105              WASHER; LOCK; MED PATT            4.0 EA       No

1/4IN KATLINK STYLE; SST

..3     23116              WASHER;FLAT;.281 IDx.625          4.0 EA       No

OD. x .065 THK;SST

.2      113741             ENVELOPE,BOILER CERTIFI           1.0 EA

CATION

.2      14618              NUT;HEX;1/4-20;300 SST;           6.0 EA       No

FULL FINISHED

.2      14618              NUT;HEX;1/4-20;300 SST;           2.0 EA       No

FULL FINISHED

.2      14618              NUT;HEX;1/4-20;300 SST;           2.0 EA       No

FULL FINISHED

.2      14618              NUT;HEX;1/4-20;300 SST;           1.0 EA       No

FULL FINISHED

.2      14661              NUT;COMP. FTG;1/4 TUBE            1.0 EA

;BRASS

.2      19288              SCREW;1/4-20x3/4;HEX HD;          6.0 EA       No

ZINC PLATED; GRADE 8

.2      19288              SCREW;1/4-20x3/4;HEX HD;          2.0 EA       No

ZINC PLATED; GRADE 8

.2      20338              TERMINAL,RING,INSUL,#10           1.0 EA

STUD,22-18AWG

.2      20338              TERMINAL,RING,INSUL,#10           1.0 EA

STUD,22-18AWG

.2      20351              TERMINL SLIPON FULL-INSU          1.0 EA

.032X.25 TAB; 22-18 AWG

.2      20351              TERMINL SLIPON FULL-INSU          1.0 EA

.032X.25 TAB; 22-18 AWG

.2      20351              TERMINL SLIPON FULL-INSU          4.0 EA

.032X.25 TAB; 22-18 AWG

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

.2      20351              TERMINL SLIPON FULL-INSU          1.0 EA

.032X.25 TAB; 22-18 AWG

.2      20351              TERMINL SLIPON FULL-INSU          1.0 EA

.032X.25 TAB; 22-18 AWG

.2      20351              TERMINL SLIPON FULL-INSU          1.0 EA

.032X.25 TAB; 22-18 AWG

.2      20351              TERMINL SLIPON FULL-INSU          1.0 EA

.032X.25 TAB; 22-18 AWG

.2      20351              TERMINL SLIPON FULL-INSU          2.0 EA

.032X.25 TAB; 22-18 AWG

.2      20351              TERMINL SLIPON FULL-INSU          2.0 EA

.032X.25 TAB; 22-18 AWG

.2      20351              TERMINL SLIPON FULL-INSU          2.0 EA

.032X.25 TAB; 22-18 AWG

.2      20352              TERMINAL FULL INS;22-18           1.0 EA

GA WIRE .020 X .187 TAB

.2      20352              TERMINAL FULL INS;22-18           1.0 EA

GA WIRE .020 X .187 TAB

.2      20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

.2      20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

.2      20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

.2      23105              WASHER; LOCK; MED PATT            6.0 EA       No

1/4IN KATLINK STYLE; SST

.2      23105              WASHER; LOCK; MED PATT            2.0 EA       No

1/4IN KATLINK STYLE; SST

.2      23105              WASHER; LOCK; MED PATT            2.0 EA       No

1/4IN KATLINK STYLE; SST

.2      23105              WASHER; LOCK; MED PATT            0.0 EA       No

1/4IN KATLINK STYLE; SST

.2      23105              WASHER; LOCK; MED PATT            1.0 EA       No

1/4IN KATLINK STYLE; SST

.2      23116              WASHER;FLAT;.281 IDx.625          6.0 EA       No

OD. x .065 THK;SST

.2      23116              WASHER;FLAT;.281 IDx.625          2.0 EA       No

OD. x .065 THK;SST

.2      23116              WASHER;FLAT;.281 IDx.625          3.0 EA       No

OD. x .065 THK;SST

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

.2      23116              WASHER;FLAT;.281 IDx.625          1.0 EA       No

OD. x .065 THK;SST

.2      23137              WASHER;LOCK;INT.TOOTH;            2.0 EA       No

1/4IN;SST

.2      234121400          WIRE;Cu;18GAx14.00 LG             1.0 EA Yes

;200 DEG C;BLACK

..3     23412              WIRE; Cu; 18-16/30             1.1667 FT

200 C; BLACK

.2      234131900          WIRE;Cu;18GAX19.00LG              1.0 EA Yes

200 DEG C; RED

..3     23413              WIRE;Cu;TYPE SRML;200          1.5833 FT

DEG. C;18 AWG;16/30;RED

.2      234183700          WIRE;Cu;18GAx37.00 LG             1.0 EA Yes

;WHITE W/ GRAY TRACER

..3     23418              WIRE; Cu-AWM-18-16/30          3.0833 FT

105 C;WHITE W/GRAY TRACE

.2      234234100          WIRE;Cu;18GAx41.00LG              1.0 EA Yes

;WHITE W/ RED TRACER

..3     23423              WIRE; Cu-AWM-18-16/30         3.14667 FT

105 C;WHITE W/RED TRACER

.2      234243800          WIRE;Cu;18GAx38.00 LG             1.0 EA Yes

;WHITE

..3     23424              WIRE; Cu-AWM-18-16/30          3.1667 FT

105 C;WHITE PLASTIC COVE

.2      260AJD             INSTRUCTIONS;LABEL;PCSM           0.0 EA

.2      260AXL             INSTRCTION;ASSY;STM COIL          0.0 EA

;LARGE

.2      260AXV             INSTRUCTION;36PCSM;LGE            0.0 EA

STEAM COIL

.2      300302             BOX;ELECT;STEAM COIL LG;          1.0 EA Yes

WITH COMPONENTS

..3     02599              BUSHING;INSULATING;               2.0 EA

3/8 X 1/2  HEYCO # 2110

..3     02600              BUSHING;INSULATOR;                1.0 EA

11/16ID

..3     06195              FTG.;COMP.3/8Tx1/4MPT             1.0 EA

..3     06196              FTG;COMP.3/8Tx1/4MPT 90'          1.0 EA

..3     100732             LABEL;PRESS SWITCH;WARN           1.0 EA

10 PSI OPERATION

..3     101873             NUT;HEX;LOCK;THIN;1/4-20          2.0 EA       No

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

;PLTD;NYLON INSERT

..3     105243             CONNECTOR;GRNFLD;3/8 IN           3.0 EA

STRAIGHT;SNAPTITE STYLE

..3     105245             CONNECTOR;GRNFLD;3/8 IN           1.0 EA

X 90 DEG;SNAPTITE STYLE

..3     105247             CONNECTOR;GREENFIELD;1/2          1.0 EA

90D;SNAPTITE STYLE

..3     105966             RELAY;DPDT-W/MTG TABS             1.0 EA

120V;50/60HZ;AC

..3     106124             SCREW;10-32x1.250;PHIL;           1.0 EA       No

PAN HD;THRD FRMNG;Zn PLT

..3     106127             SCREW;6-32x1/2;TORX/PAN           4.0 EA       No

HD;THRD FORMING;Zn PLTD

..3     106127             SCREW;6-32x1/2;TORX/PAN           2.0 EA       No

HD;THRD FORMING;Zn PLTD

..3     106128             SCREW;6-32 X 1;TORX/PAN           1.0 EA       No

HD;THRD FORMING;ZN

..3     106754             SCREW;6-32X3/8;HEX/SLOT-          2.0 EA       No

WASHER HD;SELF TAPPING

..3     108644             SCREW;CARRIAGE1/4-20x1/2          2.0 EA       No

,SST (RND HEAD; SQ NECK)

..3     108734             NUT;U TYPE(EXTRUDED)              1.0 EA       No

#10-24 UNC-2B THREAD

..3     111274             ELEC. BOX; STEAM COIL;            1.0 EA

LARGE

...4    RS01425            .060in, ALUMINIZED STEEL       0.0357 EA

48 X 120 SHEET

..3     111280             BRACKET;PRESSURE SWITCH;          1.0 EA

STEAM COIL; LARGE

...4    100781             SHEET 16GA(.060) T140          0.0125 EA

COATED STEEL 48 X 120

..3     12330              LUG GROUND 10-14GA WIRE           1.0 EA

SOLDERLESS

..3     14674              NUT;HEX;6-32;ZINC PLTD            2.0 EA       No

..3     14880              LABEL;PRESSURE SWITCH;            1.0 EA

HIGH LIMIT

..3     14881              LABEL;PRESSURE SWITCH;            1.0 EA

OPERATING

..3     19148              SCREW;HEX WASHER HD;SLT           1.0 EA       No

TYPE AB;#10X.50;ZINC

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

..3     19947              SWITCH;PRESS. CNTRL               2.0 EA

..3     20478              TIMER;SOLID STATE INTERV          1.0 EA

3 MINUTE

..3     23111              WASHER; LOCK; INTERNAL            2.0 EA       No

TOOTH;#6; ZINC PLTD

..3     23198              CONTROL;WATER LEVEL               1.0 EA

NCC#LNC-NS210-120

..3     44164              X,TERMINAL BLOCK ASSY,4           1.0 EA Yes

POLE,GAS UNITS

...4    02192              BLOCK;TERMINAL;4TERMINAL          1.0 EA

SCREWS-BR NICKEL PLTD

...4    20325              TERMINAL, CONNECTOR MALE          1.0 EA

.032X.25 ST FOR TERMINAL

...4    20326              TERMINAL CONECTOR MALE T          3.0 EA

.032X.25 45 DEG FOR TERM

...4    20327              TERMINAL CNNECTOR MALE T          4.0 EA

.032X.25 90DEG FOR TERM.

..3     52499              BRACKET;PRESS.SWITCH              1.0 EA

RETAINING

...4    RS00321            ASMESA240-304 #4 SPV-1       0.000814 EA

.060in 36 X 120 SS SHEET

.2      300309             INSTRUCTION;PT TO PT;             0.0 EA

STEAM COIL; ELECTRIC BOX

.2      3003091            INSTRUCTION;PT TO PT;LGE          0.0 EA

STM COIL;TDS OPTION

.2      699671             SUPPORT;GEN. MTG;                 2.0 EA

900MM & 1050MM STEAMCOIL

..3     RS00420            .075in - 430- #4 SPV-1          0.031 EA

BLUE 48 X 120 SHEET

.2      SM8                STEAM GENERATOR                   1.0 EA

..3     KE02415            GEN. ASS'Y/APPROVAL; SM8          1.0 EA Yes

...4    100330             GASKET;HANDHOLE;                  1.0 EA

GENERATOR

...4    FA40000-5          WELD STUD; 1/4-20-2AX1CD          2.0 EA     D No

NON-FLANG SS

...4    KE55517            OUTER SHELL  SM-8                 1.0 EA

....5   RP00266            ASME SA106 GR B 12 NOM-           1.0 EA

STD-SCH 0 375 WALL SMLS

...4    KE55522            FRONT PLATE  SM-8                 1.0 EA

....5   RS01375            ASME-SA 516 GR70 CARBON         0.055 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

0.312" TH 48X96 PL

...4    KE55527            STRIP FOR STEAM GENERATR          2.0 EA

....5   RS01171            ASME SA516 GR 70 HR             0.008 EA

7 GA; 0.187"TH 48X 96

...4    KE55529            PLATE- REAR STEAM COIL            1.0 EA

GENERATOR (SM8 0)

....5   RS01375            ASME-SA 516 GR70 CARBON         0.055 EA

0.312" TH 48X96 PL

...4    KE95007-5          NATL BOARD PLATE 50 PSI           1.0 EA

....5   KE95007-1R         NAT'L.BOARD REPAIR PLATE          1.0 EA

.....6  RS00191            ASMESA240-304 #4 SPV-1          0.003 EA

.036in, 48 X 120SS SHEET

...4    S03607             ASME SA105 HALF COUPLING          1.0 EA

1/2-14NPT,3000LBs

...4    S03609             ASME SA105 HALF COUPLING          2.0 EA

3/4-14NPT,3000LBs

...4    S03613             ASME SA105 HALF COUPLING          1.0 EA

1"NPT,3000LBs

...4    S03619             ASME SA105 HALF COUPLING          2.0 EA

3/8-18NPT CARBON STEEL

...4    S100329            ASME SA285 GRADE C COVER          1.0 EA

HANDHOLE, GENERATOR

....5   100329             COVER;HANDHOLE WLDMT;             1.0 EA

GENERATOR;

...4    S100331            ASME SA106 GRADE C RING;          1.0 EA

HANDHOLE, GENE

....5   RP00240            ASME SA106 GR.B 4" NOM          0.125 FT

SCH 80 SEAMLESS CAR.PIPE

...4    S100332            ASME SA106 GRADE C YOKE;          1.0 EA

HANDHOLE, GENE

...4    S100333            ASME SA194 GR.8NUT;HEAVY          1.0 EA

HEX;3/4-10UNC-2B SST

...4    S100863            ASME SA181 WELD FLANGE            2.0 EA

1/2-14NPT CARBON STEEL

...4    S100865            ASME SA181 WELD FLANGE            1.0 EA

3/8-18 NPT CARBON STEEL

...4    S101079            ASME SA240 BAFFLE ANGLE           1.0 EA

SM8

....5   RS00341            ASMESA240-304 .060in 2B         0.021 EA

48 X 120 SS SHEET

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

...4    S101109            PIPE BUNDLE ASS'Y; SM8            1.0 EA Yes

....5   S101104            180 DEG FITTING; SM8              1.0 EA

.....6  S101104-1          CENTRE PIECE;180DEG FTNG          1.0 EA

SM8

......7 RT00158            ASME SA213-304,SEAMLESS          0.23 FT

1-1/2ODX.065 WALL

.....6  S101104-2          END PIECE;180DEG FITTING          2.0 EA

SM8

......7 RT00158            ASME SA213-304,SEAMLESS          0.23 FT

1-1/2ODX.065 WALL

....5   S101110            3-PIPE COIL ASS'Y; SM8            2.0 EA Yes

.....6  S101102            PIPE  BRASS; SM8                  3.0 EA

......7 RP00245            ASMESB43-1 1/4NOM SCH40          1.74 FT

SEAMLESS RED BRASS PIPE

.....6  S101104            180 DEG FITTING; SM8              2.0 EA

......7 S101104-1          CENTRE PIECE;180DEG FTNG          1.0 EA

SM8

......8 RT00158            ASME SA213-304,SEAMLESS          0.23 FT

1-1/2ODX.065 WALL

......7 S101104-2          END PIECE;180DEG FITTING          2.0 EA

SM8

......8 RT00158            ASME SA213-304,SEAMLESS          0.23 FT

1-1/2ODX.065 WALL

....5   S101111            2-PIPE COIL SUB ASSY,SM8          1.0 EA Yes

.....6  S101102            PIPE  BRASS; SM8                  2.0 EA

......7 RP00245            ASMESB43-1 1/4NOM SCH40          1.74 FT

SEAMLESS RED BRASS PIPE

.....6  S101104            180 DEG FITTING; SM8              2.0 EA

......7 S101104-1          CENTRE PIECE;180DEG FTNG          1.0 EA

SM8

......8 RT00158            ASME SA213-304,SEAMLESS          0.23 FT

1-1/2ODX.065 WALL

......7 S101104-2          END PIECE;180DEG FITTING          2.0 EA

SM8

......8 RT00158            ASME SA213-304,SEAMLESS          0.23 FT

1-1/2ODX.065 WALL

....5   SM50001            NIPPLE  SM 7 5                    2.0 EA

.....6  RP00090            ASMESA312-304-1 1/4NOM          0.483 FT

SCH40 WELD. PIPE/180GRIT

...4    S101773            PROBE BAFFLE; SM8                 1.0 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

....5   RS00341            ASMESA240-304 .060in 2B         0.017 EA

48 X 120 SS SHEET

...4    SM50000            STAY BAR SM7 5                    1.0 EA

....5   RB00341            ASME SA36 3/4 DIA-HOT             2.5 FT

ROLLED CARBON STEEL BAR

1       102726             PALLET;SHIPPING;900MM             1.0 EA       No

.2      107984             PROTECTOR;STRAPPING;              4.0 EA

CORNER ANGLE

.2      110301             3x4x44 ROUGH GREEN HARD-          2.0 EA

WOOD

.2      110305             SCREW NAIL;2-7/8x.105DIA         60.0 EA       No

RND HD;BLUNT-DIA;PNT;GLV

.2      12540              4/4x6x40.5 SOUTHERN              10.0 EA

YELLOW PINE

1       102731             UPPER ASY;PCL;FINAL               1.0 EA Yes

.2      100105             X,PLUMBING ASSY,UPPER             2.0 EA Yes

,PCL

..3     00934              SEALANT; PIPE DOPE;ONE            0.0 EA

PINT CAN,RECTORSEAL #5

..3     02549              BUSHING;CONDENSER;CONV.           1.0 EA

STMR.1/2"X1/4X1/8

...4    RB01210            7/8HEX YELLOW BRASS BAR         0.078 FT

..3     02594              BUSHING;HEX;3/8x1/4;BR            1.0 EA

..3     06188              FTG.;COMP.TEE 1/4Tx1/4x           1.0 EA

1/4MPT;BR.

..3     06194              FTG;COMP.1/4Tx3/8MPT 90'          1.0 EA

..3     06216              FTG.;COMP.1/4Tx1/4MPT             1.0 EA

..3     100223             TEE;1 IN X 1/2 IN X 1 IN          1.0 EA

BLACK

..3     101394             FTG.;COMP;TEE;1/8MPT X            1.0 EA

1/4T X 1/4T

..3     101924             NIPPLE1.00NPTx 5.50               1.0 EA

SCH 40;BLK IRN TBE

..3     102078             TUBE;FORMED;PINCH VALVE           1.0 EA Yes

/DRAIN;PCL

...4    00728              TUBING;Cu; SEAMLESS.25          0.708 FT

ODX.028W-10'SOFT STR

..3     104464             VALVE;PINCH;1 IN                  1.0 EA

..3     14304              NIPPLE .25NPTxCLOSE SCH           1.0 EA

40; BRASS TBE

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

..3     14369              NIPPLE1.00NPTxCLOSE; SCH          1.0 EA

40; BLK IRON TBE

..3     14553              NOZZLE;SPRAY;DRAIN MANIF          1.0 EA

;GD1-1/8 GG;FULL JET;BRS

..3     15462              ORIFICE;FLOW CONTROLLER;          1.0 EA

1/8 GPM

..3     20245              TEE;MALE BRANCH;1/4;BR            1.0 EA

..3     21306              UNION; BLACK; 1IN; #150           1.0 EA

STANDARD

..3     565191             FTG;HOSE;1.00 Hx1 NPT             1.0 EA

; W-1/8 HOLE

...4    14481              FTG;HOSE;1HX1IN NPT;              1.0 EA

(SILVER COLOR)

...4    14481              FTG;HOSE;1HX1IN NPT;              1.0 EA

(SILVER COLOR)

.2      100315             UPPER ASY;PCL HINGE               1.0 EA Yes

HL

..3     00944              TAPE;DUCT 3 IN x60YD              1.0 RL

SLVR CLOTH

..3     06197              FTG.;COMP.3/8Tx1/4FPT             2.0 EA

..3     06214              FTG;COMP.1/4Tx1/4MPT 90'          2.0 EA

..3     100096             FACEPLATE/BODY;ASY.;PCL           1.0 EA Yes

...4    03603              COUPLING;HALF;1/4 IN;             2.0 EA

304 SST

...4    03620              COUPLING;HALF;1/2 IN;             2.0 EA

304 SST

...4    100838             CHANNEL; BODY TOP; PCL            2.0 EA

....5   101228             SHEET 10GA(.135) 304L           0.025 EA

SST 2B FIN 48 X 120

...4    100843             PL.;TOP FILLER;PCL                1.0 EA

....5   101228             SHEET 10GA(.135) 304L          0.0074 EA

SST 2B FIN 48 X 120

...4    100906             PIN-RACK GUIDE                    2.0 EA

PCL STEAMER

....5   RB00060            ASMESA479-304-3/8DIA SS         0.104 FT

SMOOTH TURNED BAR

...4    101934             BAR;HNGE CNTER;POLISHED           1.0 EA

PCL-PCS

....5   107007             CENTER HINGE BAR;STM.             1.0 EA

CONV PRO.

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

...4    107005             BRACKET; TOP HINGE                1.0 EA

STEAM PRO - CONV PRO

....5   RS01305            ASMESA240-304                  0.0069 EA

.240 .015, 48" X 96"

...4    107006             BRACKET;BOTTOM HINGE              1.0 EA

STEAM PRO - CONV PRO

....5   RS01305            ASMESA240-304                  0.0069 EA

.240 .015, 48" X 96"

...4    107008             PLATE;HINGE;CENTER                1.0 EA

STEAM PRO-CONV PRO

....5   RS01305            ASMESA240-304                  0.0085 EA

.240 .015, 48" X 96"

...4    20000              STUD;WELD;1/4-20X3/8LG            2.0 EA

CD

...4    40824              WLDMNT; BODY COMP.-PCL            2.0 EA Yes

HL

....5   03628              COUPLING;HALF;3/8IN;304           1.0 EA

SST;MUST BE 9/16 IN LNG

....5   100906             PIN-RACK GUIDE                    2.0 EA

PCL STEAMER

.....6  RB00060            ASMESA479-304-3/8DIA SS         0.104 FT

SMOOTH TURNED BAR

....5   100907             PIN-RACK GUIDE-BOT FRT            1.0 EA

PCL STEAMER

....5   69282              SHELL LOWER BODY;PCL              1.0 EA

;HINGED LEFT

.....6  101228             SHEET 10GA(.135) 304L           0.182 EA

SST 2B FIN 48 X 120

....5   69283              SHELL UPPER BODY;PCL              1.0 EA

;HINGED LEFT

.....6  101228             SHEET 10GA(.135) 304L           0.182 EA

SST 2B FIN 48 X 120

....5   69334              PIN;SHOULDER;0.250DIA x           1.0 EA

0.093

....5   69630              SPACER  STAND-OFF                 6.0 EA

SST (OHIO PART)

....5   70405              TAB BODY WLDMNT;                  2.0 EA

FAB PURPOSES ONLY

.....6  RS00430            ASMESA240-304 2B               0.0004 EA

.075in 48 X 120 SS SHEET

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

...4    42756              DRAIN;MANIFOLD;WLDMNT;            2.0 EA Yes

PCL

....5   03619              COUPLING;HALF;3/8IN;ASME          1.0 EA

SA-105;FRGED STL;BLR RTD

....5   03620              COUPLING;HALF;1/2 IN;             1.0 EA

304 SST

....5   100060             ANGLE;HOLE/DRAIN BODY;            1.0 EA

PCL

.....6  101228             SHEET 10GA(.135) 304L        0.003486 EA

SST 2B FIN 48 X 120

....5   100061             ANGLE;DRAIN TUBE;PCL              2.0 EA

.....6  101228             SHEET 10GA(.135) 304L          0.0006 EA

SST 2B FIN 48 X 120

....5   100062             PL.OUTLET;PCL                     1.0 EA

.....6  101228             SHEET 10GA(.135) 304L        0.001494 EA

SST 2B FIN 48 X 120

....5   100063             PL.;SPRAY MTG.;PCL                1.0 EA

.....6  101228             SHEET 10GA(.135) 304L        0.001494 EA

SST 2B FIN 48 X 120

....5   100065             ANGLE;DRAIN BODY                  1.0 EA

.....6  101228             SHEET 10GA(.135) 304L          0.0035 EA

SST 2B FIN 48 X 120

....5   101925             NIPPLE1.00NPTx 2.00               1.0 EA

SCH 40;SST TOE

...4    52542              BAFFLE; STEAM; J; STMR.           2.0 EA

(VJI)

....5   101469             SHEET 16GA(.060) 304 SST        0.004 EA

; 2B FIN 48 X 120

...4    52873              PLATE;COMPARTMENT BRACE;          2.0 EA

PCL(MAKES 2 FROM PRINT)

....5   RS00710            ASMESA240 304/L 2B              0.036 EA

.135in 48 X 120 SHEET

...4    53125              CHANNEL;SIDE BELLY BAND;          2.0 EA

PCL

....5   RS00710            ASMESA240 304/L 2B              0.333 EA

.135in 48 X 120 SHEET

...4    53763              CHANNEL;COMPT MTG PCL &           1.0 EA

CONV PRO

....5   RS00710            ASMESA240 304/L 2B              0.033 EA

.135in 48 X 120 SHEET

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

...4    56214              FTG.;COMPARTMENT                 10.0 EA

...4    66658              PLATE REAR PCL 304 SST            1.0 EA

....5   111307             SHEET;10 GA.;(.135) 304        0.1667 EA

SST;2B FINISH;60 x 120

...4    66660              FACEPLATE COMPT PCL 304           1.0 EA

SST

....5   101228             SHEET 10GA(.135) 304L          0.3333 EA

SST 2B FIN 48 X 120

...4    69238              BODY;STEAM SEPERATOR;PCL          2.0 EA

STEAMER

....5   00651              SHEET 12GA(.105) 304 SST       0.0666 EA

; 2B FIN 36 X 120 SA-240

...4    69334              PIN;SHOULDER;0.250DIA x          16.0 EA

0.093

...4    69335              PIN- SHOULDER                    16.0 EA

25 DIA X  218 LG

...4    69968              UPPER BODY;STM SEPERATOR          2.0 EA

PCL

....5   00651              SHEET 12GA(.105) 304 SST       0.0057 EA

; 2B FIN 36 X 120 SA-240

..3     101790             INSUL.;REAR WALL;38x29            1.0 EA Yes

PCL/CEM16.

...4    005094000          INSULATION;DUCTWRAP             0.024 RL

1.5 IN; 40 IN x100'

..3     101791             INSUL.COMPT.-BETWEEN;PCL          2.0 EA Yes

10 x 27

...4    005091000          INSUL;DUCT WRAP;1-1.5 IN        0.023 EA

X 10 IN W X 100'

..3     101792             INSUL.,COMPLETE AREA-PCL          2.0 EA Yes

8 1/2 x 13

...4    005092200          INSULATION;DUCTWRAP            0.0103 EA

1.5 IN;22 IN x100'

..3     101793             INSUL.;COMPT.WRAP-PCL             1.0 EA Yes

10FT-3IN x 32 IN

...4    005092200          INSULATION;DUCTWRAP             0.103 EA

1.5 IN;22 IN x100'

..3     102668             LATCH ASY;STM PRO                 2.0 EA Yes

...4    02095              BEARING; FLANGE; .750 ID          2.0 EA

SINTERED BRONZE

...4    100605             SCREW;SOCKET HEAD CAP             2.0 EA       No

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

;3/8-16X 1.250 LONG;SST

...4    100710             SPRING; CAM ARM RETURN            1.0 EA

...4    1044871            ARM,DOOR LATCH CAM,               1.0 EA

ASSY,PCL

....5   101265             PIN;SPRING LATCH;PCL              1.0 EA

....5   104487             CAM ARM;DOOR LATCH                1.0 EA

PCL/CONV PRO

.....6  RS01305            ASMESA240-304                   0.016 EA

.240 .015, 48" X 96"

....5   S52532             BUSHING  LATCH (OHIO)             1.0 EA

(SPECIAL ORDER)

.....6  RB00240            1 1/2DIA 303SS BAR              0.125 FT

SMOOTH TURNED

...4    14665              NUT;HEX;LOCK;1/4-20;SST;          1.0 EA       No

NYLON INSERT

...4    14675              NUT;HEX;3/8-16;CAD PLTD.          2.0 EA       No

...4    18306              RING;RETAINING;EXTERNAL           1.0 EA

WALDES #5100-75-H

...4    450821             PL,WLDMNT,LATCH,PCL,HL            1.0 EA

....5   100604             BLOCK-ADJ SCREW                   2.0 EA

PCL/PCS

.....6  RB00120            3/4DIA  303SS BAR               0.052 FT

C'TRELESS GR(.746-.748)

....5   100605             SCREW;SOCKET HEAD CAP             2.0 EA       No

;3/8-16X 1.250 LONG;SST

....5   101283             STUD;WELD;1/4-20X1/2 IN           1.0 EA

CD SST;NO FLANGE

....5   101405             PIN;SPRING&BUMPER;PCL             1.0 EA

....5   104542             BRACKET;SOLENOID MTG;PCL          1.0 EA

.....6  00651              SHEET 12GA(.105) 304 SST       0.0074 EA

; 2B FIN 36 X 120 SA-240

....5   14675              NUT;HEX;3/8-16;CAD PLTD.          2.0 EA       No

....5   66095              PIN- PAWL PIVOT                   1.0 EA

PCL/PCS STEAMER

....5   66650              PL.;LATCH MTG.                    1.0 EA

.....6  666501             SHEET 7GA(.180) 304 SSTL          0.1 EA

; 2B FIN 16.375 X 48.00

....5   66673              PIN- LATCH PIVOT                  1.0 EA

PCL/PCS STEAMER

.....6  RB00223            1 1/4 DIA -303 S/S ROD          0.218 FT

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

SMOOTH TURNED

..3     106236             X,HANDLE ASSY,PCL STMR            2.0 EA

...4    1068661            HANDLE;PLASTIC;STEAM/             1.0 EA

CONV-PRO

...4    106908             STUD;HANDLE STEAM/CONV.           1.0 EA

PRO

..3     18309              RING;RETAINING;EXTERNAL           2.0 EA

1/2IN DIA SHAFT

..3     19129              SCREW;HEX;3/8-16x7/8;            12.0 EA       No

PLTD;ALLOY STEEL

..3     23101              WASHER;LOCK MEDIUM               18.0 EA       No

PATTRN;3/8IN; ZINC PLTD

..3     23120              WASHER;FLAT;3/8ID;7/8OD          12.0 EA       No

.064/.104 THK;ZINC PLTD

..3     23150              WASHER;LOCK;EXTRNL TOOTH          9.0 EA       No

3/8IN ZNC PLTD

..3     41137              DOOR;ASY.;STEAMPRO                2.0 EA Yes

...4    02096              BEARING;OIL IMPREGNATED           1.0 EA

.753 ID;POROUS BRONZE

...4    100546             DOOR WELDMT  PCL REVE             1.0 EA

...4    100557             SCREW;SOCKET HD;SST;              2.0 EA       No

1/4-28x1

...4    101788             SHIM;DR;PCL;16GA.                 2.0 EA

....5   RS00341            ASMESA240-304 .060in 2B         0.009 EA

48 X 120 SS SHEET

...4    101818             COVER ASY;DR-CONVPRO;PCL          1.0 EA Yes

....5   100759             BAR;DR HANDLE;PCL                 1.0 EA

.....6  RB00691            1" +0.040"BLK EXTR.ACRYL          1.3 FT

BAR PROTECTED GLOSS FINI

....5   100990             SPRING; WAVE; PCL                 2.0 EA

....5   101044             BRACKET;DOOR HANDLE               2.0 EA

....5   101048             PAD; INSULATOR; PLASTIC           2.0 EA

(DOOR BRACKET)

....5   19288              SCREW;1/4-20x3/4;HEX HD;          4.0 EA       No

ZINC PLATED; GRADE 8

....5   23105              WASHER; LOCK; MED PATT            4.0 EA       No

1/4IN KATLINK STYLE; SST

....5   23116              WASHER;FLAT;.281 IDx.625          4.0 EA       No

OD. x .065 THK;SST

....5   54230              WLDMNT;COVER DOOR PCL             1.0 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

STEAMER

.....6  112659             COVER;DOOR;PCL STEAMER            1.0 EA

FORMED

......7 RS00476            ASMESA240-304 #4 SPV-1           0.25 EA

.075in 48 X 120 SS SHEET

...4    104292             GASKET;DOOR-PCL CUT 84            1.0 EA

IN; ENDS GLUED TOGETHER

...4    107507             ADHESIVE;LOCTITE;BLUE;            0.0 EA       No

THRDLKER 242;50ML;#24231

...4    109420             SCREW;SET;CUP POINT; HEX          1.0 EA       No

SOCKET;1/4-20 X .375;SST

...4    109823             PIN;ROLLER;PCL/CONV PRO           1.0 EA

...4    109826             RING;RETAINING;SPIRAL;            1.0 EA

.750 DIA. SHAFT;SST

...4    19123              SCREW;SOCKET HD;                  4.0 EA       No

1/4-28X1.5;SST

...4    23098              WASHER;.75IDx1.1880Dx.05          3.0 EA       No

S/S

...4    52061              BAR;HINGE PIVOT                   1.0 EA

...4    68084              ROLLER- DOOR LATCH                1.0 EA

PCL/PCS STEAMER

....5   S68084             ROLLER- DOOR LATCH                1.0 EA

PCL/PCS STEAMER

.....6  RB00240            1 1/2DIA 303SS BAR              0.066 FT

SMOOTH TURNED

...4    WR50193            PIN;SPRING;1/4 IN X 1IN           1.0 EA       No

LG;SST

...4    WR50193            PIN;SPRING;1/4 IN X 1IN           1.0 EA       No

LG;SST

..3     441481             PAWL;WLDMNT;PCL;H.L.              2.0 EA

...4    100855             PAWL;BAR;PCL                      1.0 EA

....5   00424              BAR;.25X1;304;SST               0.469 FT

....5   RB00730            1/4X 1-304-HRAP A-276-            0.5 FT

STAINLESS STEEL BAR

...4    100856             PAWL;ANGLE;PCL                    1.0 EA

....5   00403              ANGLE;1/8X1X1;304SST            0.083 FT

...4    52540              BUSHING;PAWL PIVOT;               1.0 EA

STEAMPRO/CONVPRO

...4    66079              LOCK;PAWL LATCH                   1.0 EA

....5   RS01305            ASMESA240-304                   0.014 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

.240 .015, 48" X 96"

..3     45099              DOOR HARDWARE                     1.0 EA Yes

...4    02093              BEARING -.566 ID .753 OD          4.0 EA

POROUS BRONZE

...4    100004             BAR;NUT;(3)3/8-16 HOLES;          3.0 EA

PCL

...4    100539             WASHER; DOOR HINGE                4.0 EA       No

...4    101231             SCREW; 10-24 X 0.500 SST          4.0 EA       No

PHILLIPS TRUSS HEAD

...4    101935             BAR;HINGE BOTTOM                  1.0 EA

....5   100001             BAR- HINGE BOTTOM                 1.0 EA

(HINGED LEFT)

.....6  RB00790            1/2X 1 1/2-304-HRAP               0.7 FT

A276 S S  FLAT TRUE BAR

....5   RB00790            1/2X 1 1/2-304-HRAP               0.6 FT

A276 S S  FLAT TRUE BAR

...4    101936             BAR;HINGE TOP                     1.0 EA

....5   100000             BAR- HINGE TOP                    1.0 EA

(HINGED LEFT)

.....6  RB00790            1/2X 1 1/2-304-HRAP               0.7 FT

A276 S S  FLAT TRUE BAR

....5   RB00790            1/2X 1 1/2-304-HRAP               0.6 FT

A276 S S  FLAT TRUE BAR

...4    19134              SCREW;3/8-16x1;HH;SST             6.0 EA       No

...4    53124              CHANNEL;DOOR END;PCL              2.0 EA

POLISHED;J STMR

....5   RS00476            ASMESA240-304 #4 SPV-1         0.0294 EA

.075in 48 X 120 SS SHEET

.2      100768             CYLINDER;PRESSURE LOCK            2.0 EA

.2      100771             SOLENOID;CONT DUTY;CLASS          2.0 EA

F; INSUL;120VAC;60HZ

.2      100883             FTG.;COMPRESSION;TEE              2.0 EA

3/8Tx1/4NPTx3/8T;BRASS

.2      100914             BRACKET;CYLINDER MNTG             2.0 EA

..3     00651              SHEET 12GA(.105) 304 SST        0.001 EA

; 2B FIN 36 X 120 SA-240

.2      101012             PIN;SOLENOID;PCL;.125             2.0 EA

DIA' AISI TYPE 304 SST

.2      101013             STRAP;SOLENOID;PCL                2.0 EA

..3     101013F            STRAP;SOLENOID;SHEARED            1.0 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

...4    101469             SHEET 16GA(.060) 304 SST     0.000518 EA

; 2B FIN 48 X 120

.2      101015             SPRING;LATCH;PCL                  2.0 EA

.2      101061             MANIF;WATER;ASY;PCL               1.0 EA Yes

..3     00934              SEALANT; PIPE DOPE;ONE            0.0 EA

PINT CAN,RECTORSEAL #5

..3     03050              CAP;PIPE;ROUND;1/2 IN;            1.0 EA

BRASS

..3     06214              FTG;COMP.1/4Tx1/4MPT 90'          4.0 EA

..3     100976             MANIF.WATER;VALVE SPPT            1.0 EA

;PCL

...4    RP00320            1/2SCH 80 RED BRASS              1.45 FT

PIPE

..3     14304              NIPPLE .25NPTxCLOSE SCH           2.0 EA

40; BRASS TBE

..3     14411              NIPPLE .25NPTx 1.50; SCH          2.0 EA

40; BRASS TBE

..3     21303              UNION; 1/2IN; BRASS               1.0 EA

..3     22236              VALVE; SOLENOID; 3 WAY.;          2.0 EA

60 HZ.

..3     22241              VLVE;SOLENOID-2 WAY NC 1          2.0 EA

T; ORIFICE 5/32 120V/60

.2      101062             COMPARTMENT;CONDENSER AS          2.0 EA Yes

Y(PCL)

..3     02549              BUSHING;CONDENSER;CONV.           1.0 EA

STMR.1/2"X1/4X1/8

...4    RB01210            7/8HEX YELLOW BRASS BAR         0.078 FT

..3     02594              BUSHING;HEX;3/8x1/4;BR            1.0 EA

..3     05235              ELBOW;STREET;90 DEG.;             1.0 EA

1/4 IN; BR.;LONG PATTERN

..3     05244              ELBOW;90 DEG.;3/8 IN;BR           1.0 EA

..3     06193              FTG.;COMP.1/4Tx3/8MPT             1.0 EA

..3     100109             NOZZLE;SPRAY                      1.0 EA

..3     100982             VALVE;PRESSURE RELIEF             1.0 EA

..3     14304              NIPPLE .25NPTxCLOSE SCH           1.0 EA

40; BRASS TBE

.2      101794             VALVE;STEAM;ASY;PCL               2.0 EA Yes

..3     00934              SEALANT; PIPE DOPE;ONE            0.0 EA

PINT CAN,RECTORSEAL #5

..3     02566              BUSHING;HEX;3/4x1/2;BR            1.0 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

..3     05231              ELBOW;STREET;3/4x90;BR            1.0 EA

FORGED STYLE ONLY

..3     06240              FTG;HOSE BARB;3/4x3/4MPT          1.0 EA

BR.

..3     102362             ELBOW;UNION;MALE TO FEM           1.0 EA

1/2IN;BRASS

..3     14324              NIPPLE .50NPTxCLOSE; SCH          1.0 EA

40; BRASS TBE

..3     22193              VALVE; SOLENOID 3/4X3/4;          1.0 EA

120V; 60 HZ

.2      102074             TUBING;Cu-3/8X21.25LG             2.0 EA Yes

-FORMED

..3     00729              TUBE;Cu;SEAMLESS; .375          2.563 FT

x.032WALL 10'SOFT STR

.2      102077             TUBING-FORMED (LO COMP-           2.0 EA Yes

STM TR/DRAIN FIT)-PCL

..3     00728              TUBING;Cu; SEAMLESS.25          0.583 FT

ODX.028W-10'SOFT STR

.2      102078             TUBE;FORMED;PINCH VALVE           2.0 EA Yes

/DRAIN;PCL

..3     00728              TUBING;Cu; SEAMLESS.25          0.708 FT

ODX.028W-10'SOFT STR

.2      102079             TUBING-FORMED UP COMP-            1.0 EA Yes

VALVE/DRAIN -PCL

..3     00728              TUBING;Cu; SEAMLESS.25          1.167 FT

ODX.028W-10'SOFT STR

.2      102080             TUBING-FORMED (COMPART-           2.0 EA Yes

MENT/STM TRAP) - PCL

..3     00728              TUBING;Cu; SEAMLESS.25          1.792 FT

ODX.028W-10'SOFT STR

.2      102081             TUBING-FORMED LO COMP-            1.0 EA Yes

VALVE/DRAIN-PCL

..3     00728              TUBING;Cu; SEAMLESS.25          0.875 FT

ODX.028W-10'SOFT STR

.2      102082             TUBING-FORMED (UP COMP-           1.0 EA Yes

VALVE/FL RESTRICTOR)-PCL

..3     00728              TUBING;Cu; SEAMLESS.25          1.396 FT

ODX.028W-10'SOFT STR

.2      102082             TUBING-FORMED (UP COMP-           1.0 EA Yes

VALVE/FL RESTRICTOR)-PCL

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

..3     00728              TUBING;Cu; SEAMLESS.25          1.396 FT

ODX.028W-10'SOFT STR

.2      102083             TUBING-FORMED LO COMP-            1.0 EA Yes

VALVE/FL RESTOR - PCL

..3     00728              TUBING;Cu; SEAMLESS.25          1.833 FT

ODX.028W-10'SOFT STR

.2      102085             TUBING-FORMED CONDENSER           2.0 EA Yes

BOX/VALVE - PCL

..3     00728              TUBING;Cu; SEAMLESS.25          1.833 FT

ODX.028W-10'SOFT STR

.2      104452             TUBING;COPPER SEAMLESS            1.0 EA Yes

.375X17LG.

..3     00729              TUBE;Cu;SEAMLESS; .375          1.417 FT

x.032WALL 10'SOFT STR

.2      104453             TUBING;Cu SEAMLESS                1.0 EA Yes

.375X.032WALLX11.75LG

..3     00729              TUBE;Cu;SEAMLESS; .375          0.979 FT

x.032WALL 10'SOFT STR

.2      14618              NUT;HEX;1/4-20;300 SST;           8.0 EA       No

FULL FINISHED

.2      14661              NUT;COMP. FTG;1/4 TUBE            4.0 EA

;BRASS

.2      14674              NUT;HEX;6-32;ZINC PLTD           12.0 EA       No

.2      146771             NUT; ACORN; 1/4-20; SST;          2.0 EA       No

THREADLOCK (LOW CROWN)

.2      18313              O-RING;005 TRADE SIZE;            4.0 EA

EP;7/64IDx15/64ODx1/16W

.2      19259              SCREW,RD HD, 6-32X1/2             8.0 EA

.2      19288              SCREW;1/4-20x3/4;HEX HD;          8.0 EA       No

ZINC PLATED; GRADE 8

.2      20559              TRAP;THERMOSTATIC 1/4IN           2.0 EA

COMP X 1/4IN COMP

.2      23105              WASHER; LOCK; MED PATT            8.0 EA       No

1/4IN KATLINK STYLE; SST

.2      23111              WASHER; LOCK; INTERNAL           12.0 EA       No

TOOTH;#6; ZINC PLTD

.2      23116              WASHER;FLAT;.281 IDx.625         16.0 EA       No

OD. x .065 THK;SST

.2      45093              RACK,STEAM TUBE,MTG.HDW.          1.0 EA

.2      69298              SCREEN;DRAIN COMPARTMENT          2.0 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

(STAINLESS)

..3     RS00341            ASMESA240-304 .060in 2B        0.0048 EA

48 X 120 SS SHEET

1       105065             PNL;REAR;900;AL CTD ST            1.0 EA

BASE;30 HIGH

.2      107396             SHEET 20GA(.036) T140            0.25 EA

COATED STEEL 48 X 120

1       105182             LABEL;CALIFORNIA;WARNING          1.0 EA

FIBERGLASS;CALIF;PROP 65

1       36PCSM01A          COMMON PARTS;36PCSM               1.0 EA Yes

.2      01024              STRAP;2200'ROLL;.031x5/8        0.029 RL       No

PPY9x8CORE;24COILS/PALET

.2      03100              CATCH; MAGNETIC                   2.0 EA

.2      03204              CLAMP;WORM DRIVE;.812-            2.0 EA

1.50 DIA.

.2      03204              CLAMP;WORM DRIVE;.812-            4.0 EA

1.50 DIA.

.2      03207              CLAMP;BAND;WORM DRIVE;            2.0 EA

MAX OD 1.125

.2      03606              COUPLING;FULL;1/2 IN;BR.          1.0 EA

.2      06237              FTG;HOSE BARB;1/2Hx               1.0 EA

1/2MPT;BR

.2      085112250          HOSE;EPDM;3/4 ID                  1.0 EA Yes

22.5 LG

..3     08511              HOSE;WHITE EPDM;3/4 IN          1.875 FT

ID;FDA APPROVED,150 PSI

.2      085113000          HOSE;EPDM;3/4 IDx30               1.0 EA Yes

..3     08511              HOSE;WHITE EPDM;3/4 IN            2.5 FT

ID;FDA APPROVED,150 PSI

.2      085114050          HOSE;EPDM;3/4 ID                  1.0 EA Yes

40.5 LG

..3     08511              HOSE;WHITE EPDM;3/4 IN          3.375 FT

ID;FDA APPROVED,150 PSI

.2      085152800          HOSE; REINF. SILICONE;            1.0 EA Yes

1/2 IDx28 LG

..3     08515              HOSE;1/2 INID;REINFORCED     2.333333 FT

SILICON

.2      100144             SEAT;1/2 IN BANDING               4.0 EA       No

.2      100194             X,TUBE ASSY,STM,UP RIGHT          4.0 EA Yes

,LOW LEFT,PCL

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

..3     03262              CLIP;STEAMTUBE;PCL                1.0 EA

...4    RS00430            ASMESA240-304 2B               0.0007 EA

.075in 48 X 120 SS SHEET

..3     192771             SCREW;FILLISTER HD;               1.0 EA       No

5/16-18X1/2;SST W/SEALNT

..3     69800              TUBE - STEAM; PCL                 1.0 EA

...4    RT00040            3/8OD X  035WALL-304           1.7143 FT

S/S SEAMLESS TUBING

.2      100196             X,TUBE ASSY,UP LEFT,              4.0 EA Yes

LOW.RIGHT,PCL

..3     100184             CLIP;STEAM TUBE;PCL;30            1.0 EA

...4    RS00430            ASMESA240-304 2B                0.003 EA

.075in 48 X 120 SS SHEET

..3     192771             SCREW;FILLISTER HD;               1.0 EA       No

5/16-18X1/2;SST W/SEALNT

..3     69800              TUBE - STEAM; PCL                 1.0 EA

...4    RT00040            3/8OD X  035WALL-304           1.7143 FT

S/S SEAMLESS TUBING

.2      100198             X,TUBE ASSY,STEAM,CENTER          2.0 EA Yes

..3     100581             CLIP;SUPPORT;CENTER TUBE          1.0 EA

...4    RS00430            ASMESA240-304 2B                0.003 EA

.075in 48 X 120 SS SHEET

..3     192771             SCREW;FILLISTER HD;               1.0 EA       No

5/16-18X1/2;SST W/SEALNT

..3     69800              TUBE - STEAM; PCL                 1.0 EA

...4    RT00040            3/8OD X  035WALL-304           1.7143 FT

S/S SEAMLESS TUBING

.2      100273             SHIM;COMPARTMENT MNTG             1.0 EA

;PCL FRONT

..3     RS01425            .060in, ALUMINIZED STEEL        0.021 EA

48 X 120 SHEET

.2      1002731            SHIM;COMPARTMENT;PCL;             2.0 EA

FRONT;SHORT

..3     RS01425            .060in, ALUMINIZED STEEL       0.0062 EA

48 X 120 SHEET

.2      100275             SHIM;COMPARTMENT MTG              4.0 EA

;PCL

..3     RS01425            .060in, ALUMINIZED STEEL       0.0078 EA

48 X 120 SHEET

.2      100728             WASHER; TROUGH DRAIN -            1.0 EA       No

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

TABLE-TOP

.2      1015631            SCREW;HEX HD;5/16-18;             8.0 EA       No

1.25;GRADE 5 ZINC PLTD

.2      102541             SUPPORT;BETWEEN DOOR-PCL          1.0 EA

.2      102544             SUPPORT;TTOP/COMP PCL-39          2.0 EA

.2      1025441            SUPPORT;TTOP/COMP PCL 57          1.0 EA

.2      1048074            CONSOLE ASY;900MM;(2)             1.0 EA Yes

SWITCH CTRL;BLR BASE

..3     06246              FTG;COMP.3/8Tx1/8FPT;BR.          1.0 EA

..3     07168              GAUGE;PRESSURE;BACK MTD;          1.0 EA

15PSI OPERTION;W/ARTWORK

..3     102535             ANGLE;MAGNET MTG;BASES;           1.0 EA

W/TWO DOORS

...4    100781             SHEET 16GA(.060) T140          0.0024 EA

COATED STEEL 48 X 120

..3     105134             CONSOLE;WELDMT;900MM (2)          1.0 EA Yes

SWITCH CTRL;SCREW MTD

...4    105114             CONSOLE;BASE 900 MM;(2)           1.0 EA

CH CTRL

....5   00661              SHEET 16GA(.060) 304 SST       0.0303 EA

; #3 FIN 36 x 125 PVC

...4    19896              STUD;WELD;1/4-20X5/8IN            2.0 EA

LONG CD;SST

...4    19918              STUD;WLD;STYLE F;FLANGED          2.0 EA

10-24X3/8LG;304 SST

..3     106740             BOX;SWITCH PROTECTION;            1.0 EA Yes

STD;BASES

...4    106740F            BOX;SWITCH PROTECTION             1.0 EA

....5   RS00872            .036in ALUMINIZED SHEET         0.021 EA

48 X 120 SHEET

..3     108482             PLUG;HOLE;0.312 IN DIA.           1.0 EA

..3     111971-1           LABEL, CONSOLE SWITCH             1.0 EA

..3     14665              NUT;HEX;LOCK;1/4-20;SST;          2.0 EA       No

NYLON INSERT

..3     14692              NUT;HEX;LOCK;10-24;PLTD;          2.0 EA       No

NYLON INSERT

..3     19993              SWITCH;RCKR;DPDT;ON/OFF           1.0 EA

RED LT;15A;125V

..3     19994              SWITCH;ROCKER;DPDT;AMBR           1.0 EA

LGHT;MOM;SEE DWG

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

..3     300317             WIRE ASM;WHT/RED;TRACER;          1.0 EA Yes

20315; 6  LG

...4    20315              TERMINAL,PGGYBAK SLIPON           2.0 EA

TAB,.032x.25 INS 22-15

...4    23423              WIRE; Cu-AWM-18-16/30             0.5 FT

105 C;WHITE W/RED TRACER

.2      105243             CONNECTOR;GRNFLD;3/8 IN           2.0 EA

STRAIGHT;SNAPTITE STYLE

.2      1052481900         CONDUIT;GREENFIELD; 3/8           1.0 EA Yes

x19 LG

..3     105248             CONDUIT;ELEC;GREENFIELD        1.5833 FT

;3/8 IN;FLEXIBLE

.2      105280             STRAINER;TABLETOP DRAIN/          1.0 EA

HOSE FITTING

.2      1054692200         HOSE; EPDM; BLACK WATER           1.0 EA Yes

1 ID x 22.00 LG

..3     105469             HOSE;EPDM;BLK;WATER;1 IN        1.833 FT

ID x1-7/16 IN OD

.2      1054693900         HOSE;EPDM;1.00 ID                 1.0 EA Yes

39 LG

..3     105469             HOSE;EPDM;BLK;WATER;1 IN         3.25 FT

ID x1-7/16 IN OD

.2      105561             BUSHING;ANTISHORT;3/8 IN          2.0 EA

TRADE SIZE #1

.2      106141             LABEL;PATENTS                     1.0 EA

.2      106698             SCREW;TRUSS HD PHILLIPS;          4.0 EA       No

10-24X1 1/2;SST

.2      106736             TAIL PIECE, DRAIN                 1.0 EA

.2      107312             CLAMP;HOSE;1 1/8 IN OD;           4.0 EA       No

METAL TENSION BAND

.2      108734             NUT;U TYPE(EXTRUDED)              4.0 EA       No

#10-24 UNC-2B THREAD

.2      14423              NIPPLE .50NPTx 2.25; SCH          1.0 EA

40;BRASS;THREAD PER DWG.

.2      14605              NUT; LOCK;1/2IN PIPE;BAR          1.0 EA

K; 5 THREAD; 119BL;BRASS

.2      14652              NUT;HEX;5/16-18;SST               8.0 EA       No

.2      14674              NUT;HEX;6-32;ZINC PLTD            2.0 EA       No

.2      18358              RIVET;FLUSH BREAK;1/8IN          12.0 EA       No

DIA;.126-.187 GRIP; SST

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

.2      19206              SCREW;6-32x1;RD HD SLOT           2.0 EA       No

;ZINC PLTD

.2      19441              CARTON,W/COVER,41"X41.5"          1.0 EA

X70" (REF 900 MM UNITS

.2      19555              SPACER;1/4x1/2x0.812 LG.          4.0 EA

;ALUM.

.2      19874              STRAINER;DUPLEX#1134 FOR          1.0 EA

FOR DRAIN BOWL;2CP UNIV

.2      20351              TERMINL SLIPON FULL-INSU          4.0 EA

.032X.25 TAB; 22-18 AWG

.2      23106              WASHER;LOCK;MED PAT;5/16          8.0 EA       No

;SST

.2      23134              WASHER;FLAT;5/16ID 3/4OD         16.0 EA       No

1/16 THK;ZINC PLTD

.2      234065500          WIRE;Cu;18GA;WH/BLK               1.0 EA Yes

TCR 55 LG

..3     23406              WIRE; Cu; AWM18-16/30           4.583 FT

105C; WHITE W/BLK TRACE

.2      234245500          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

55 LG

..3     23424              WIRE; Cu-AWM-18-16/30           4.583 FT

105 C;WHITE PLASTIC COVE

.2      260AJD             INSTRUCTIONS;LABEL;PCSM           0.0 EA

.2      260AKQ             MOUNTING CONCEPT;CONSOLE          0.0 EA

TO BASE; DOUBLE HINGE

.2      260APB             INSTALLATION;USE;CARE             1.0 EA

INSTR(STM PRO STEAMERS

.2      260APE             RECRD KEEPING(SVC OR              1.0 EA       No

MNTNC SHT)ALL STMR PROD

.2      260APH             CUSTOMER DIRECTORY;CLEVE          1.0 EA       No

SLS REP/'MAINT/REP CNTRS

.2      260BBQ             INST;RTG PLT;SM SERIES            0.0 EA

STEAM COIL BASES

.2      260BKT             INSTALL MANUAL                    1.0 EA

ALL CLASSICS

.2      260BKW             OPERATOR MANUAL                   1.0 EA

CLASSIC CONVECTION

.2      412182             X,BRKT ASSY,TOP HINGE             1.0 EA Yes

LEFT SIDE

..3     16312              PIN;HINGE;MOD.BASE;SHORT          1.0 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

;S/S

..3     528611             BRACKET;TOP HINGE;L.S.            1.0 EA

.2      412192             X,BRKT,ASSY,TOP HINGE,            1.0 EA

RIGHT SIDE

..3     16312              PIN;HINGE;MOD.BASE;SHORT          1.0 EA

;S/S

..3     528621             BRACKET;TOP HINGE;R.S.            1.0 EA

...4    101228             SHEET 10GA(.135) 304L           0.002 EA

SST 2B FIN 48 X 120

.2      442283             LABEL-PACKAGE;PRO;STEAM           1.0 EA Yes

COIL

..3     05511              ENVELOPE;PACKING LIST;            1.0 EA

SMALL;4 1/2IN X 5 1/2IN

..3     102551             LABEL;BOILER WARRANTY             1.0 EA

..3     104350             LABEL;SERVICE HOTLINE             1.0 EA

NATIONAL SERVICE COOP

..3     104527             LABEL;SANITATION-U.L.;            1.0 EA

NSF; STANDARD NO. 4

..3     105901             LABEL;NAMEPLATE;                  1.0 EA

CLEVELAND

..3     106087             LABEL;CAUTION:                    1.0 EA

DO NOT LIFT

..3     111535             LABEL; WARNING POWER              1.0 EA

DISCONNECT

..3     112250             LABEL;WATER CONNECTION;           1.0 EA

NSF 4;STRNRS INSTALLED

..3     113062             MYLAR;SERIAL TAG COVER            1.0 EA

4" X 6"

..3     113062             MYLAR;SERIAL TAG COVER            1.0 EA

4" X 6"

..3     14818              LABEL;115 VOLTS ONLY              1.0 EA

..3     14834              LABEL;HANDHOLE PLATE              1.0 EA

;SERVICE

..3     14889              LABEL;DRAIN OUTLET TAG            1.0 EA

..3     14919              LABEL;CAUTION; DOORS AND          1.0 EA

HANDLES MAY BE HOT

..3     14923              LABEL;STEAM CONNECTION;           1.0 EA

3/4" IPS

..3     15020              LABEL;STEAM GENERATOR;            1.0 EA

MAINTENANCE

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

..3     300311             LABEL;WIRING;ST COIL LG;          1.0 EA

W OPT INT BLOWDOWN

.2      532741             BRACKET;MAGNET MTNG/CAM           1.0 EA

ALUM.

..3     100781             SHEET 16GA(.060) T140          0.0013 EA

COATED STEEL 48 X 120

.2      70088              TABLETOP;900MM H.L.PCL            1.0 EA Yes

..3     70088F             TABLETOP;900MM H.L. PCL           1.0 EA

...4    00661              SHEET 16GA(.060) 304 SST        0.333 EA

; #3 FIN 36 x 125 PVC

.2      FA05002-34         O-RING; EPDM-E50 (A-012)         10.0 EA       No

1       66721              PNL;REAR COVER;430SST;            1.0 EA

PCL

.2      102591             SHEET 20GA(.036) 430 SST        0.333 EA

; #3 FIN 48 X 132 PVC

1       C00102             FLANGED; 6 IN LEG QTY 4           1.0 EA Yes

.2      06162              FOOT;ADJ.;FLANGED 6 IN            4.0 EA

; TRI-BOLT HOLE PATTERN

.2      108204             SCREW;HEX HD LAG;ZINC             4.0 EA       No

PLTD;5/16X2 IN LG.

.2      14668              NUT;HEX JAM;3/4-10;ZINC-          4.0 EA       No

PLTD;FINISHED; GRADE 2

.2      23136              WASHER;FLAT;3/8ID;7/8OD           4.0 EA       No

1/16IN THK SST

1       C053               SHEETING;430S/S;CONV/STM          1.0 EA Yes

PRO;UPPER BODY

.2      100131             PNL;LFT SIDE SHEET;PCL            1.0 EA

430 S/S

..3     RS00187            .036in, 430 #4 BLUE-1            0.25 EA

48 X 120 SS SHEET

.2      100132             PNL;RT SIDE SHEETING              1.0 EA

PCL;430SST

..3     RS00187            .036in, 430 #4 BLUE-1            0.25 EA

48 X 120 SS SHEET

.2      100697             PNL;TOP;WLDMNT;PCL                1.0 EA Yes

S/S

..3     19920              STUD;WELD;1/4 DIAX1/4LG           2.0 EA

SST;NO THRD;CD;NO FLANGE

..3     66722              PNL TOP COVER PCL 304SS           1.0 EA

...4    RS00139            .036in - 430- #4 BLUE-1        0.3333 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

36 X 120 SS SHEET

.2      101737             PNL;PEDESTAL;SIDE;WELD;           2.0 EA

PCL;430 SST

..3     100126             PNL;PEDESTAL SIDE;430             1.0 EA

...4    RS00187            .036in, 430 #4 BLUE-1           0.015 EA

48 X 120 SS SHEET

..3     16338              WELD PIN; .190 DIA; S/S-          2.0 EA

.2      1044741            X,PEDS ASSY,PCL,430 SST           1.0 EA Yes

..3     104474             PNL;PED FRNT;STM.PRO/             1.0 EA

CONV.PRO;430 SST

...4    RS00187            .036in, 430 #4 BLUE-1          0.1111 EA

48 X 120 SS SHEET

..3     19918              STUD;WLD;STYLE F;FLANGED          4.0 EA

10-24X3/8LG;304 SST

..3     19928              STUD;WELD;6-32x1/2 S/S;           6.0 EA

FULL FLANGED;CD

1       C055               STD RACKS;CONV/STM PRO            1.0 EA Yes

.2      102003             RACK;ASSY.;CENTER WIRE;           2.0 EA

PCL

.2      102420             RACK;ASSY.;RIGHT SIDE;            2.0 EA

PCL/CONV. PRO

.2      102421             RACK;ASSY.;LEFT SIDE              2.0 EA

;PCL/CONV. PRO

1       C124               FRAME; SST; STD 36;               1.0 EA Yes

PCL; CONV. PRO

.2      1020662            HINGE;LWR DOOR ASY;L.H.           1.0 EA Yes

..3     104550             SUPPORT;LOWER DH                  1.0 EA

EXTENDEDW/SLOT;SST

...4    RS00710            ASMESA240 304/L 2B              0.003 EA

.135in 48 X 120 SHEET

..3     16312              PIN;HINGE;MOD.BASE;SHORT          1.0 EA

;S/S

.2      1020663            HINGE;LWR DOOR ASY.R.H.           1.0 EA Yes

..3     104550             SUPPORT;LOWER DH                  1.0 EA

EXTENDEDW/SLOT;SST

...4    RS00710            ASMESA240 304/L 2B              0.003 EA

.135in 48 X 120 SHEET

..3     16312              PIN;HINGE;MOD.BASE;SHORT          1.0 EA

;S/S

.2      111248             BASE ASY-WLDMNT;SST               1.0 EA Yes

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

900 MM;PCL STEAMER

..3     100244             CHANNEL SUPPORT PCL 304;          2.0 EA

600/900 BASE

...4    RS00710            ASMESA240 304/L 2B             0.0166 EA

.135in 48 X 120 SHEET

..3     100389             FRAME; UPPER BASE;                1.0 EA Yes

ASSY;PCL

...4    53728              CROSSMEMBR.;UPR.REAR;FOR          2.0 EA

900MM BASE; 304 S/S

....5   RS00711            ASME SA240-304/304L-2B           0.02 EA

.135in 60 X 120 SHEET

...4    53730              CROSSMEMBER;UPPER RIGHT           2.0 EA

& LEFT;SST

....5   RS00710            ASMESA240 304/L 2B              0.017 EA

.135in 48 X 120 SHEET

..3     1042141            SUPPORT; PRESS.SWITCH             1.0 EA

BRK'T BOILER BASE ASS'Y

...4    RS00430            ASMESA240-304 2B               0.0023 EA

.075in 48 X 120 SS SHEET

..3     1042881            UPRIGHT;FRONT; R&L; U             2.0 EA

SST

...4    RS00710            ASMESA240 304/L 2B             0.0156 EA

.135in 48 X 120 SHEET

..3     40865              SUPPORT ASY;WLDMNT;               2.0 EA

BOILER;S/S

...4    51164              ANGLE;GEN. SUPPORT;SST            1.0 EA

....5   RS00710            ASMESA240 304/L 2B             0.0175 EA

.135in 48 X 120 SHEET

...4    76150              PLATE;BOILER SUPPORT ANG          2.0 EA

....5   RS00711            ASME SA240-304/304L-2B        0.00075 EA

.135in 60 X 120 SHEET

..3     42722              FRAME ASY;BOTTOM;S/S;900          1.0 EA Yes

MM

...4    53716              ANGLE;1/8X2X2X33.969;SST          2.0 EA

;LOWER FRAME FRONT/REAR

....5   RS00710            ASMESA240 304/L 2B              0.028 EA

.135in 48 X 120 SHEET

...4    53718              ANGLE;1/8X2X2X30.812;SST          2.0 EA

;LOWER FRAME LEFT/RIGHT

....5   RS00710            ASMESA240 304/L 2B              0.028 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

.135in 48 X 120 SHEET

..3     511631             UPRIGHT;PLAIN;U; SST              2.0 EA

...4    RS00710            ASMESA240 304/L 2B             0.0125 EA

.135in 48 X 120 SHEET

..3     53296              CLIP;SIDE&REAR BASE PNL           2.0 EA

S/S PANEL;S/S

...4    RS00711            ASME SA240-304/304L-2B         0.0002 EA

.135in 60 X 120 SHEET

..3     53298              CLIP;GEN. HOLD DOWN               2.0 EA

...4    RS00711            ASME SA240-304/304L-2B         0.0005 EA

.135in 60 X 120 SHEET

..3     53766              BRACE CENTER UPPER PCL            2.0 EA

SST

...4    RS00711            ASME SA240-304/304L-2B         0.0417 EA

.135in 60 X 120 SHEET

..3     57627              GUSSET;4 HOLES;S/S BASE           1.0 EA

...4    RS00430            ASMESA240-304 2B                0.002 EA

.075in 48 X 120 SS SHEET

..3     77601              GUSSET;PLAIN;                     5.0 EA

13GA.SST ALL BASES

...4    RS00430            ASMESA240-304 2B               0.0045 EA

.075in 48 X 120 SS SHEET

.2      14618              NUT;HEX;1/4-20;300 SST;           2.0 EA       No

FULL FINISHED

.2      19288              SCREW;1/4-20x3/4;HEX HD;          2.0 EA       No

ZINC PLATED; GRADE 8

.2      23105              WASHER; LOCK; MED PATT            2.0 EA       No

1/4IN KATLINK STYLE; SST

.2      23116              WASHER;FLAT;.281 IDx.625          2.0 EA       No

OD. x .065 THK;SST

1       C180               STD;430S/S;DOORS;36 UNIT          1.0 EA Yes

.2      110593             DOOR,SWING,ASSY,450MM/            2.0 EA Yes

900MM BASE

..3     110594             DOOR; BASE;450MM/900MM            1.0 EA

BASE

...4    RS00187            .036in, 430 #4 BLUE-1          0.0833 EA

48 X 120 SS SHEET

..3     18357              RIVET;FLUSH BREAK;1/8IN           4.0 EA       No

DIA;.062-.125 GRIP; SST

1       MCS008             NOT USED MECH CTRLS;              1.0 EA Yes

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

STEAMPRO

.2      104967             LABEL;LADDER DIA;MECH.            1.0 EA

CONT.PCL

.2      107259             CONTROL MECHANICAL;PNL.;          1.0 EA Yes

LOWER COMPT.;STEAM PRO

..3     03524              RELAY;120V;50/60HZ;AC             1.0 EA

DPDT

..3     03525              SOCKET;RELAY                      1.0 EA

..3     104305             BRACKET;MICRO SWITCH MTG          1.0 EA

STM PRO

...4    00696              SHEET 18GA(.048) 304 SST        0.002 EA

; #3 FIN 30 X 96 PVC

..3     104319             BRACKET;MOUNTING;MICRO            1.0 EA

SWITCH;STM PRO

...4    RS00430            ASMESA240-304 2B               0.0016 EA

.075in 48 X 120 SS SHEET

..3     104403             MICRO SWITCH                      2.0 EA

..3     104740             WIRE ASY;18GA;5 LG;WH/            1.0 EA Yes

PNK;W/2  20356

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234210500          WIRE; Cu; #18 AWG; 5              1.0 EA Yes

LG;WH/PINK TRACER

....5   23421              WIRE; Cu-AWM-18-16/30           0.417 FT

105 C;WHITE W/PINK TRACE

..3     104750             WIRE ASY.; #18 AWG; 14LG          1.0 EA Yes

WH/YEL W/ 20364 & 20365

...4    20364              TERMINAL RING INSUL #10           1.0 EA

STUD 22-18AWG;TAPE ROLL

...4    20365              TERMINAL;BUTT CONNECTOR           1.0 EA

F.I.;22-16;TAPE ROLL

...4    234251400          WIRE;Cu;18GA;WH/YLW;              1.0 EA Yes

TRACER;14.00 LG

....5   23425              WIRE; Cu-AWM-18-16/30           1.167 FT

WHITE W/YELLW TRACER UL1

..3     104751             WIRE ASY;18GA;7 LG;WHT/           1.0 EA Yes

BLK;W/TWO 20364

...4    20364              TERMINAL RING INSUL #10           2.0 EA

STUD 22-18AWG;TAPE ROLL

...4    234060700          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

BLACK TCR; 7.00 LG

....5   23406              WIRE; Cu; AWM18-16/30           0.583 FT

105C; WHITE W/BLK TRACE

..3     104752             WIRE ASY;18GA;4 ;WHITE;           1.0 EA Yes

W/ 20315 & 20365

...4    20315              TERMINAL,PGGYBAK SLIPON           1.0 EA

TAB,.032x.25 INS 22-15

...4    20365              TERMINAL;BUTT CONNECTOR           1.0 EA

F.I.;22-16;TAPE ROLL

...4    234240400          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

4 LG

....5   23424              WIRE; Cu-AWM-18-16/30           0.333 FT

105 C;WHITE PLASTIC COVE

..3     104753             WIRE ASY;18GA;16 IN;WHT/          1.0 EA Yes

ORANGE W/20364 & BARE

...4    20364              TERMINAL RING INSUL #10           1.0 EA

STUD 22-18AWG;TAPE ROLL

...4    20364              TERMINAL RING INSUL #10           1.0 EA

STUD 22-18AWG;TAPE ROLL

..3     104755             WIRE ASY;18GA;16 LG;WH/           1.0 EA Yes

BLK;W/ONE 20364

...4    20364              TERMINAL RING INSUL #10           1.0 EA

STUD 22-18AWG;TAPE ROLL

...4    234061600          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

;BLACK TCR; 16.00 LG

....5   23406              WIRE; Cu; AWM18-16/30        1.333333 FT

105C; WHITE W/BLK TRACE

..3     104757             WIRE ASY;18GA;12 ;WHT/            1.0 EA Yes

PINK W/20356

...4    20356              TERMINL SLIPON FULL INS;          1.0 EA

22-18AWG.032X.25 FEMALE

...4    234211200          WIRE; Cu; #18 AWG; 12             1.0 EA Yes

WH/PINK TRACER

....5   23421              WIRE; Cu-AWM-18-16/30             1.0 FT

105 C;WHITE W/PINK TRACE

..3     1051880500         WIRE ASM.;18GA; WH/BLACK          1.0 EA Yes

5.00 LG; 20356/20364

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    20364              TERMINAL RING INSUL #10           1.0 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

STUD 22-18AWG;TAPE ROLL

...4    234060500          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

/BLACK TCR; 5.00 LG

....5   23406              WIRE; Cu; AWM18-16/30           0.417 FT

105C; WHITE W/BLK TRACE

..3     1051881100         WIRE ASM.;18GA; WH/BLACK          1.0 EA Yes

20356/20364; 11.00 LG

...4    20356              TERMINL SLIPON FULL INS;          1.0 EA

22-18AWG.032X.25 FEMALE

...4    20364              TERMINAL RING INSUL #10           1.0 EA

STUD 22-18AWG;TAPE ROLL

...4    234061100          WIRE; Cu; 18GA. WHITE             1.0 EA Yes

BLACK TCR; 11.00 LG

....5   23406              WIRE; Cu; AWM18-16/30           0.917 FT

105C; WHITE W/BLK TRACE

..3     1051970900         WIRE ASM.;18GA;WHITE;             1.0 EA Yes

20315;20356;9 LG

...4    20315              TERMINAL,PGGYBAK SLIPON           1.0 EA

TAB,.032x.25 INS 22-15

...4    20356              TERMINL SLIPON FULL INS;          1.0 EA

22-18AWG.032X.25 FEMALE

..3     1051971200         WIRE ASM.;18GA; WHITE;2           1.0 EA Yes

20356;12 LG

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234241200          WIRE;Cu;18GA;WHITE;               1.0 EA Yes

12 LG

....5   23424              WIRE; Cu-AWM-18-16/30             1.0 FT

105 C;WHITE PLASTIC COVE

..3     1051971900         WIRE ASM.;18GA; WHITE;2           1.0 EA Yes

20356;19 LG

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234241900          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

19 LG

....5   23424              WIRE; Cu-AWM-18-16/30           1.583 FT

105 C;WHITE PLASTIC COVE

..3     1053440700         WIRE ASM.18GA;WH;20315/           1.0 EA Yes

20352; 7 LG

...4    20315              TERMINAL,PGGYBAK SLIPON           1.0 EA

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

TAB,.032x.25 INS 22-15

...4    20352              TERMINAL FULL INS;22-18           1.0 EA

GA WIRE .020 X .187 TAB

...4    234240700          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

7 LG

....5   23424              WIRE; Cu-AWM-18-16/30           0.583 FT

105 C;WHITE PLASTIC COVE

..3     1053441300         WIRE ASM.18GA;WHT;20315/          1.0 EA Yes

;20356;13 LG

...4    20315              TERMINAL,PGGYBAK SLIPON           1.0 EA

TAB,.032x.25 INS 22-15

...4    20356              TERMINL SLIPON FULL INS;          1.0 EA

22-18AWG.032X.25 FEMALE

...4    23424              WIRE; Cu-AWM-18-16/30           1.083 FT

105 C;WHITE PLASTIC COVE

..3     1053450500         WIRE ASM.18GA;WH/BLK;TWO          1.0 EA Yes

20356;5 LG

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234060500          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

/BLACK TCR; 5.00 LG

....5   23406              WIRE; Cu; AWM18-16/30           0.417 FT

105C; WHITE W/BLK TRACE

..3     1053450600         WIRE ASM.18GA;WH/BLK;TWO          2.0 EA Yes

20356;6 LG

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234060600          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

BLACK TCR; 6.00 LG

....5   23406              WIRE; Cu; AWM18-16/30             0.5 FT

105C; WHITE W/BLK TRACE

..3     1053450900         WIRE ASM.18GA;WH/BLK;TWO          1.0 EA Yes

20356;9 LG

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234060900          WIRE; Cu; 18GA;WHT/BLK            1.0 EA Yes

TCR; 9 LG

....5   23406              WIRE; Cu; AWM18-16/30            0.75 FT

105C; WHITE W/BLK TRACE

..3     105406             PNL;ASY;MECH;TIMER;               1.0 EA Yes

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

STM. PRO

...4    06246              FTG;COMP.3/8Tx1/8FPT;BR.          1.0 EA

...4    100277             GAUGE; DUAL;10PSI VAC/PR          1.0 EA

(W/ARTWORK)2IN RING/BACK

...4    104993             SWITCH;ROCKER-THREE PO            1.0 EA

N;#TIGM61-1F-BL-FN

...4    105407             PNL;WLDMNT;MECH.TI                1.0 EA

STM.PRO

....5   1048302            PNL; CONTROL; MECH. CON           1.0 EA

TEAM PRO

.....6  101469             SHEET 16GA(.060) 304 SST       0.0119 EA

; 2B FIN 48 X 120

....5   19897              STUD;WELD;10-24X1.250LG           1.0 EA

CD; SST

....5   19919              STUD;WLD;STYLE F;1/4-20X          2.0 EA

7/8; 304 SST; CD

...4    112125             SCREW;M4x8;PHILLIPS;PAN-          2.0 EA       No

HD;SST

...4    115341             LABEL;CONTROL PANEL MECH          1.0 EA

TIMER;STEAMPRO XVI

...4    14692              NUT;HEX;LOCK;10-24;PLTD;          1.0 EA       No

NYLON INSERT

...4    19151              SCREW;SET CUP PT;SST              8.0 EA       No

3-48X1/8LG;SOCKET

...4    20323              TERMINAL,#10 RING/ .25            4.0 EA

MAL ZIERICK #898-200

...4    20477              TIMER; SOLID STATE;3 SEC          1.0 EA

DELAY ON BREAK

...4    300888             TIMER,60 MIN,SP/DT                1.0 EA

W/KNOB,(IFG MI2H)

...4    41350              BUZZER;ASSY;CONTINUOUS            1.0 EA Yes

DUTY;120VAC;50-60HZ

....5   300848             BUZZER,CONTINUOUS DUTY,           1.0 EA

120VAC,50-60 HZ

....5   300849             WIRE JUMPER,BUZZER                2.0 EA

(FOR 300848 BUZZER)

...4    53264              CLIP;INSTRUMENT PANEL             4.0 EA

FRAME

...4    56235              BAR;INSTRUMENT PNL FRAME          2.0 EA

TOP/BOTTOM

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

....5   RB00500            B-02026;EXTRU.ALUM                0.4 FT

CLEAR ANODIZE

...4    56236              BAR;INSTRUMENT PNL;FRAME          2.0 EA

SIDE

..3     1054081400         WIRE ASM.; 18GA RED;              1.0 EA Yes

14.00LG

...4    20356              TERMINL SLIPON FULL INS;          1.0 EA

22-18AWG.032X.25 FEMALE

...4    234131400          WIRE;Cu;18GA;RED;150              1.0 EA Yes

DEG. C;14.00 LG

....5   23413              WIRE;Cu;TYPE SRML;200           1.167 FT

DEG. C;18 AWG;16/30;RED

..3     1054111000         WIRE ASM.WHT;ONE 20315;&          1.0 EA Yes

BARE;10 LG

...4    20315              TERMINAL,PGGYBAK SLIPON           1.0 EA

TAB,.032x.25 INS 22-15

...4    23424              WIRE; Cu-AWM-18-16/30          0.8333 FT

105 C;WHITE PLASTIC COVE

..3     1060651200         WIRE ASM.18GA;WH/BL;2-            1.0 EA Yes

;20356;12 LG

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234161200          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

TCR; 12.00 LG

....5   23416              WIRE; Cu-AWM-18-16/30             1.0 FT

105 C;WHITE W/BLUE TRACE

..3     14674              NUT;HEX;6-32;ZINC PLTD            4.0 EA       No

..3     19206              SCREW;6-32x1;RD HD SLOT           4.0 EA       No

;ZINC PLTD

..3     20451              WIRE TIE;NYLON; 5.6IN LG          6.0 EA

PANDUIT PLT1.5M-M OR EQ

..3     23111              WASHER; LOCK; INTERNAL            4.0 EA       No

TOOTH;#6; ZINC PLTD

..3     44168              TERMINAL BLK ASY; 2 POLE          1.0 EA Yes

...4    02199              BLOCK;2 TERMINAL                  1.0 EA

SCREWS-BR NICKLE PLTD

...4    20325              TERMINAL, CONNECTOR MALE          2.0 EA

.032X.25 ST FOR TERMINAL

...4    20326              TERMINAL CONECTOR MALE T          2.0 EA

.032X.25 45 DEG FOR TERM

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

...4    20327              TERMINAL CNNECTOR MALE T          2.0 EA

.032X.25 90DEG FOR TERM.

.2      1072591            CONTROL MECHANICAL;PNL.;          1.0 EA Yes

UPPER COMPT.;STEAM PRO

..3     03524              RELAY;120V;50/60HZ;AC             1.0 EA

DPDT

..3     03525              SOCKET;RELAY                      1.0 EA

..3     104305             BRACKET;MICRO SWITCH MTG          1.0 EA

STM PRO

...4    00696              SHEET 18GA(.048) 304 SST        0.002 EA

; #3 FIN 30 X 96 PVC

..3     104319             BRACKET;MOUNTING;MICRO            1.0 EA

SWITCH;STM PRO

...4    RS00430            ASMESA240-304 2B               0.0016 EA

.075in 48 X 120 SS SHEET

..3     104403             MICRO SWITCH                      2.0 EA

..3     104740             WIRE ASY;18GA;5 LG;WH/            1.0 EA Yes

PNK;W/2  20356

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234210500          WIRE; Cu; #18 AWG; 5              1.0 EA Yes

LG;WH/PINK TRACER

....5   23421              WIRE; Cu-AWM-18-16/30           0.417 FT

105 C;WHITE W/PINK TRACE

..3     104742             WIRE ASY;18GA;13 IN.;WHT          1.0 EA Yes

W/20315 & 20365

...4    20315              TERMINAL,PGGYBAK SLIPON           1.0 EA

TAB,.032x.25 INS 22-15

...4    20365              TERMINAL;BUTT CONNECTOR           1.0 EA

F.I.;22-16;TAPE ROLL

...4    234241300          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

13 LG

....5   23424              WIRE; Cu-AWM-18-16/30           1.083 FT

105 C;WHITE PLASTIC COVE

..3     104750             WIRE ASY.; #18 AWG; 14LG          1.0 EA Yes

WH/YEL W/ 20364 & 20365

...4    20364              TERMINAL RING INSUL #10           1.0 EA

STUD 22-18AWG;TAPE ROLL

...4    20365              TERMINAL;BUTT CONNECTOR           1.0 EA

F.I.;22-16;TAPE ROLL

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

...4    234251400          WIRE;Cu;18GA;WH/YLW;              1.0 EA Yes

TRACER;14.00 LG

....5   23425              WIRE; Cu-AWM-18-16/30           1.167 FT

WHITE W/YELLW TRACER UL1

..3     104751             WIRE ASY;18GA;7 LG;WHT/           1.0 EA Yes

BLK;W/TWO 20364

...4    20364              TERMINAL RING INSUL #10           2.0 EA

STUD 22-18AWG;TAPE ROLL

...4    234060700          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

BLACK TCR; 7.00 LG

....5   23406              WIRE; Cu; AWM18-16/30           0.583 FT

105C; WHITE W/BLK TRACE

..3     104754             WIRE ASY;18GA;35 IN;WHT/          1.0 EA Yes

ORANGE W/ONE 20364

...4    20364              TERMINAL RING INSUL #10           1.0 EA

STUD 22-18AWG;TAPE ROLL

...4    234203500          WIRE; Cu; 18GA; WH/OR             1.0 EA Yes

TRACER; 35.00 LG

....5   23420              WIRE; Cu-AWM-18-16/30           2.917 FT

105 C;WHITE W/ORANGE TRA

..3     104756             WIRE ASY;18GA;35 LG;WH/           1.0 EA Yes

BLK;W/ONE 20364

...4    20364              TERMINAL RING INSUL #10         2.917 EA

STUD 22-18AWG;TAPE ROLL

...4    234063500          WIRE; Cu; 18GA;WHT/BLK            1.0 EA Yes

TCR; 35 LG

....5   23406              WIRE; Cu; AWM18-16/30           2.917 FT

105C; WHITE W/BLK TRACE

..3     104759             WIRE ASY;#18 AWG;29 IN.;          1.0 EA Yes

WH/PINK W/ONE 20365

...4    20365              TERMINAL;BUTT CONNECTOR           1.0 EA

F.I.;22-16;TAPE ROLL

...4    234212900          WIRE; Cu; #18 AWG. 29             1.0 EA Yes

WH/PINK TRACER

....5   23421              WIRE; Cu-AWM-18-16/30           2.417 FT

105 C;WHITE W/PINK TRACE

..3     1051880500         WIRE ASM.;18GA; WH/BLACK          1.0 EA Yes

5.00 LG; 20356/20364

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

...4    20364              TERMINAL RING INSUL #10           1.0 EA

STUD 22-18AWG;TAPE ROLL

...4    234060500          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

/BLACK TCR; 5.00 LG

....5   23406              WIRE; Cu; AWM18-16/30           0.417 FT

105C; WHITE W/BLK TRACE

..3     1051881100         WIRE ASM.;18GA; WH/BLACK          1.0 EA Yes

20356/20364; 11.00 LG

...4    20356              TERMINL SLIPON FULL INS;          1.0 EA

22-18AWG.032X.25 FEMALE

...4    20364              TERMINAL RING INSUL #10           1.0 EA

STUD 22-18AWG;TAPE ROLL

...4    234061100          WIRE; Cu; 18GA. WHITE             1.0 EA Yes

BLACK TCR; 11.00 LG

....5   23406              WIRE; Cu; AWM18-16/30           0.917 FT

105C; WHITE W/BLK TRACE

..3     1051970500         WIRE ASM;18GA;WHITE;TWO           1.0 EA Yes

20356;5 LG

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234240500          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

50 LG

....5   23424              WIRE; Cu-AWM-18-16/30           0.417 FT

105 C;WHITE PLASTIC COVE

..3     1051971200         WIRE ASM.;18GA; WHITE;2           1.0 EA Yes

20356;12 LG

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234241200          WIRE;Cu;18GA;WHITE;               1.0 EA Yes

12 LG

....5   23424              WIRE; Cu-AWM-18-16/30             1.0 FT

105 C;WHITE PLASTIC COVE

..3     1051972600         WIRE ASM.;18GA; WHITE;2           1.0 EA Yes

20356; 26 LG

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234242600          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

26 LG

....5   23406              WIRE; Cu; AWM18-16/30           2.167 FT

105C; WHITE W/BLK TRACE

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

..3     1051972700         WIRE ASM.;18GA; WHITE;2           1.0 EA Yes

20356; 27 LG

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234242700          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

27 LG

....5   23424              WIRE; Cu-AWM-18-16/30             2.5 FT

105 C;WHITE PLASTIC COVE

..3     1053440700         WIRE ASM.18GA;WH;20315/           1.0 EA Yes

20352; 7 LG

...4    20315              TERMINAL,PGGYBAK SLIPON           1.0 EA

TAB,.032x.25 INS 22-15

...4    20352              TERMINAL FULL INS;22-18           1.0 EA

GA WIRE .020 X .187 TAB

...4    234240700          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

7 LG

....5   23424              WIRE; Cu-AWM-18-16/30           0.583 FT

105 C;WHITE PLASTIC COVE

..3     1053441300         WIRE ASM.18GA;WHT;20315/          1.0 EA Yes

;20356;13 LG

...4    20315              TERMINAL,PGGYBAK SLIPON           1.0 EA

TAB,.032x.25 INS 22-15

...4    20356              TERMINL SLIPON FULL INS;          1.0 EA

22-18AWG.032X.25 FEMALE

...4    23424              WIRE; Cu-AWM-18-16/30           1.083 FT

105 C;WHITE PLASTIC COVE

..3     1053450500         WIRE ASM.18GA;WH/BLK;TWO          1.0 EA Yes

20356;5 LG

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234060500          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

/BLACK TCR; 5.00 LG

....5   23406              WIRE; Cu; AWM18-16/30           0.417 FT

105C; WHITE W/BLK TRACE

..3     1053450500         WIRE ASM.18GA;WH/BLK;TWO          1.0 EA Yes

20356;5 LG

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234060500          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

/BLACK TCR; 5.00 LG

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

....5   23406              WIRE; Cu; AWM18-16/30           0.417 FT

105C; WHITE W/BLK TRACE

..3     1053450600         WIRE ASM.18GA;WH/BLK;TWO          2.0 EA Yes

20356;6 LG

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234060600          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

BLACK TCR; 6.00 LG

....5   23406              WIRE; Cu; AWM18-16/30             0.5 FT

105C; WHITE W/BLK TRACE

..3     1053452600         WIRE ASM.18GA;WH/BLK;TWO          1.0 EA Yes

20356;26 LG

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234062600          WIRE; Cu; 18GA;WHT/BLK            1.0 EA Yes

TCR; 26 LG

....5   23406              WIRE; Cu; AWM18-16/30           2.167 FT

105C; WHITE W/BLK TRACE

..3     105406             PNL;ASY;MECH;TIMER;               1.0 EA Yes

STM. PRO

...4    06246              FTG;COMP.3/8Tx1/8FPT;BR.          1.0 EA

...4    100277             GAUGE; DUAL;10PSI VAC/PR          1.0 EA

(W/ARTWORK)2IN RING/BACK

...4    104993             SWITCH;ROCKER-THREE PO            1.0 EA

N;#TIGM61-1F-BL-FN

...4    105407             PNL;WLDMNT;MECH.TI                1.0 EA

STM.PRO

....5   1048302            PNL; CONTROL; MECH. CON           1.0 EA

TEAM PRO

.....6  101469             SHEET 16GA(.060) 304 SST       0.0119 EA

; 2B FIN 48 X 120

....5   19897              STUD;WELD;10-24X1.250LG           1.0 EA

CD; SST

....5   19919              STUD;WLD;STYLE F;1/4-20X          2.0 EA

7/8; 304 SST; CD

...4    112125             SCREW;M4x8;PHILLIPS;PAN-          2.0 EA       No

HD;SST

...4    115341             LABEL;CONTROL PANEL MECH          1.0 EA

TIMER;STEAMPRO XVI

...4    14692              NUT;HEX;LOCK;10-24;PLTD;          1.0 EA       No

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

NYLON INSERT

...4    19151              SCREW;SET CUP PT;SST              8.0 EA       No

3-48X1/8LG;SOCKET

...4    20323              TERMINAL,#10 RING/ .25            4.0 EA

MAL ZIERICK #898-200

...4    20477              TIMER; SOLID STATE;3 SEC          1.0 EA

DELAY ON BREAK

...4    300888             TIMER,60 MIN,SP/DT                1.0 EA

W/KNOB,(IFG MI2H)

...4    41350              BUZZER;ASSY;CONTINUOUS            1.0 EA Yes

DUTY;120VAC;50-60HZ

....5   300848             BUZZER,CONTINUOUS DUTY,           1.0 EA

120VAC,50-60 HZ

....5   300849             WIRE JUMPER,BUZZER                2.0 EA

(FOR 300848 BUZZER)

...4    53264              CLIP;INSTRUMENT PANEL             4.0 EA

FRAME

...4    56235              BAR;INSTRUMENT PNL FRAME          2.0 EA

TOP/BOTTOM

....5   RB00500            B-02026;EXTRU.ALUM                0.4 FT

CLEAR ANODIZE

...4    56236              BAR;INSTRUMENT PNL;FRAME          2.0 EA

SIDE

..3     1054082500         WIRE ASM.; 13GA; RED;ONE          1.0 EA Yes

BARE 25.00 LG

...4    20356              TERMINL SLIPON FULL INS;          1.0 EA

22-18AWG.032X.25 FEMALE

...4    234132500          WIRE; Cu; 18GA; RED               1.0 EA Yes

25.00 LG

....5   23413              WIRE;Cu;TYPE SRML;200           2.083 FT

DEG. C;18 AWG;16/30;RED

..3     1054112800         WIRE ASM.WHT;ONE 20315;&          1.0 EA Yes

BARE;28 LG

...4    20315              TERMINAL,PGGYBAK SLIPON           1.0 EA

TAB,.032x.25 INS 22-15

...4    234242800          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

28 LG

....5   23424              WIRE; Cu-AWM-18-16/30           2.333 FT

105 C;WHITE PLASTIC COVE

..3     1060652200         WIRE ASM.18GA;WH/BL;2-            1.0 EA Yes

Level   Component Item     Description              Quantity Per UM Ph  T Iss

------- ------------------ ------------------------ ------------ -- --- - ---

;20356; 22 LG

...4    20356              TERMINL SLIPON FULL INS;          2.0 EA

22-18AWG.032X.25 FEMALE

...4    234162200          WIRE; Cu; 18GA; WHITE             1.0 EA Yes

TCR; 22.00 LG

....5   23416              WIRE; Cu-AWM-18-16/30           1.833 FT

105 C;WHITE W/BLUE TRACE

..3     14674              NUT;HEX;6-32;ZINC PLTD            4.0 EA       No

..3     19206              SCREW;6-32x1;RD HD SLOT           4.0 EA       No

;ZINC PLTD

..3     20451              WIRE TIE;NYLON; 5.6IN LG          6.0 EA

PANDUIT PLT1.5M-M OR EQ

..3     23111              WASHER; LOCK; INTERNAL            4.0 EA       No

TOOTH;#6; ZINC PLTD

..3     44168              TERMINAL BLK ASY; 2 POLE          1.0 EA Yes

...4    02199              BLOCK;2 TERMINAL                  1.0 EA

SCREWS-BR NICKLE PLTD

...4    20325              TERMINAL, CONNECTOR MALE          2.0 EA

.032X.25 ST FOR TERMINAL

...4    20326              TERMINAL CONECTOR MALE T          2.0 EA

.032X.25 45 DEG FOR TERM

...4    20327              TERMINAL CNNECTOR MALE T          2.0 EA

.032X.25 90DEG FOR TERM.

.2      111780             WASHER;RETAINER;INSUL.            2.0 EA

..3     107396             SHEET 20GA(.036) T140          0.0009 EA

COATED STEEL 48 X 120

.2      14618              NUT;HEX;1/4-20;300 SST;           4.0 EA       No

FULL FINISHED

.2      19972              SWITCH;THERMAL CLOSE 193          2.0 EA

DEG F; OPEN 173 DEG. F

.2      23105              WASHER; LOCK; MED PATT            4.0 EA       No

1/4IN KATLINK STYLE; SST

.2      23116              WASHER;FLAT;.281 IDx.625          4.0 EA       No

OD. x .065 THK;SST