Kit Number- KE003737-1 (30 Gallon) and KE003737-2 (40 Gallon)

This kit is to convert a TR skillet (built before September 2011) from power tilt using a electric SKF actuator to manual hand tilt.

Tools Required

| - Wrenches - 7/16, 1/.2, 5/8, 11/16 - Screw Drivers - Philip, Flat - Adjustable wrench - Snap ring Pliers | - Oil 2 quarts - Silicone |
| --- | --- |

Kit parts

| Handle, cover and fasteners | Hydraulic tank assembly | Hydraulic actuator assembly | Pump assembly | Front panel assembly |
| --- | --- | --- | --- | --- |

Notes

It is recommended that after the first step the unit be moved to an open area were access is available from all sides.

Warnings

This conversion should only be done by a qualified service technician.

Preparation

| Tilt Pan. Remove safety thermostat from pan bottom Lower pan back down to its cooking position. Turn electrical and Gas supplies off. Remove connections to the unit. Move unit to an open area were you have access to all sides. Remove right panel, rear panel, open front panel |  |
| --- | --- |

Disassembly

| Disassembly and removal of electrical cover Remove screws holding electrical box cover over controls Remove orange and yellow wires going to motor Remove RTD wires #9,10 on thermostat board Remove safety thermostat |  |  |
| --- | --- | --- |
| Remove wire harness Label wires with reference to terminal block number in order to make reattachment easier. Remove wires from terminal block. Loosen liquid tight fitting, pull wires from cover. Set cover aside |  |  |
| Remove and relocate Power switch Disconnect wired from terminal block Insert power switch into new front panel (indicator light facing top of panel). |  |  |
| Remove and relocate terminal Block & thermostat Remove screws holding terminal block Remove wire from indicator light (#6) on terminal block Remove dial Remove nut from thermostat Remove screws and pull board and off. Careful not to pull thermostat shaft out of board. |  |  |
| Relocate thermostat Push on stop assembly Push shaft through hole Fasten board with 2 screws Install sealing nut on shaft | Align stop assembly |  |
| Mount terminal block Connect wires from power switch Connect wire from indicator light | Thermostat and terminal block remounted |  |
| Remove front Panel May have to loosen hinge on frame in order to allow the panel hinge to pass the leg flange |  |  |
| Remove actuator Top pin first followed by bottom pin. |  |  |
| Disassembly complete |  |  |

Reassembly

| Hydraulic Actuator Install new hydraulic ram using existing pins and new clips found in kit Bottom first then top |  |  |
| --- | --- | --- |
| Pump Assembly Remove mounting nuts and lock washers. Leave spacer in place. Mount bracket on outside of frame using bottom bolt. Do not fully tighten. Swivel into place and mount remaining 2 bolts Tighten in place |  |  |
| Hydraulic Tank Mount tank to frame, 2 bolts Connect hose from tank to pump Connect hose from Actuator to tank Connect second hose from tank to pump Fill tank with oil |  |  |
| Add Oil Turn intake elbow to up position (if not already) Carefully pour in a little oil Turn shaft 1/2 turn clockwise Add more oil Repeat a couple more times Reconnect hose Put on hand wheel and turn until oil runs out of bleed hole on Actuator |  |  |
| Bleed system Install bleed nut Raise pan up Let pan down Carefully open bleed screw until oil and air come out Turn hand wheel ½ turn clockwise to expel more air. Repeat process once more |  |  |
| Checks Check for oil leaks Check oil level in tank |  |  |
| Reinstall safety thermostat Using liquid tight fitting from original box install safety thermostat on to new cover Raise pan Add a small amount of silicone to seal hole Run bulb up through heat shield and reinstall on pan plate Lower pan Remove hand wheel |  |  |
| Front Panel Install front panel Retighten hinges Mount panel support bracket |  |  |

| Wires Mount conduit wire cable to cover Mount RTD cable to cover Connect blue and black wires from terminal block to safety thermostat Reconnect 8 wires to terminal block Connect RTD sensor wires to #9, 10 on board Connect ground wire Use tie wraps to neaten the wires |  |
| --- | --- |
| Install cover Left Side- Remove nuts holding switch guard, drop box in place and install nuts. Right side- install with hardware provided Tie wrap safety thermostat tube to RTD sensor cable |  |
| Final Assembly Install side and back panels Tilt front panel into position and install with trim piece Screw knob onto push rod Install hand wheel |  |