Tumble Chiller

With the Power on, Insert the USB drive preloaded with the HMI program into the USB port on the touch screen.

•	Select Terminal settings   File Management  Copy files   Copy Applications.

•	You want to copy the program from the USB drive to the internal memory of the panel view.  To do this select   Source: External and   Destination: Internal then select copy.

•	The program will then be saved into the Panelview, then back out to the main screen.

•	You must select “Load Application”.  Then select the program that is to be used. – select

load then yes.

•	Now you must set up the Start up options.

•	Select Terminal settings  Startup options.  “Select Run current application” for “on startup” – Then “OK”

•	The IP address must now be set for this device.  To do this select: Terminal Settings  Network and Communication   Network Connections > network adapters. Then select “IP address” With DHCP set to “no” enter an IP address of 192.168.100.2 and a subnet mask of 255.255.255.0   Then click “ok” >> next screen select “OK” – remove USB drive.

•	Go back to the main setup screen and select “Run application” Upon start up, the model number and description (ex.  60 HA-MKDL-60-CCT-RL) should display on the title screen.  Additionally, all of the buttons should be functional.  If this does not happen, there is a communication error between the Panelview and the PLC. Cycle power to see if the program loads correctly.

•	Set Time & Date.